| Subject Code 22PC1CE205 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular and Supplementary Examinations, May 2026

Structural Analysis

(CE)

Time: 3 hours                                                                  			 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | State the conditions for a perfect frame and distinguish it from a deficient frame. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | What is the double integration method for finding deflection? State its limitations. | 2M | CO-2 | BL-1 |
|  | Differentiate between a cantilever and a propped cantilever beam with a neat sketch. | 2M | CO-3 | BL-1 |
|  | Explain any two advantages of Continuous Beams. | 2M | CO-4 | BL-1 |
|  | Define an influence line diagram. How does it differ from a shear force diagram? | 2M | CO-5 | BL-2 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | Determine the forces in all the members of the truss shown in Fig.1 and indicate the magnitude and nature of the forces on the diagram of the truss. Fig.1 | 10M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2 | A truss is loaded and supported as shown in Fig.2. Determine the axial forces in the members CD, BD, CE, EG, EF & DF. Fig.2 | 10M | CO-1 | BL-4 |
| UNIT-II |  |  |  |  |
| 3 | A simply supported beam of span 6 m carries a point load of 50 kN at 2 m from the left end. If E = 200 GPa and I = 40 × 10⁻⁶ m⁴, find (i) slopes at both supports, (ii) deflection under the load, and (iii) maximum deflection and its location, using Macaulay's method. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4 | A cantilever beam of 3 m span carries a uniformly distributed load of 12 kN/m over its entire length and a concentrated load of 30 kN at the free end. Using the moment area method, determine the slope and deflection at the free end. Take EI = 8 × 10³ kN·m². | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5 | A propped cantilever beam of span 5 m is fixed at A and propped at B. It carries a uniformly distributed load of 20 kN/m over the entire span. Using the consistent deformation method, find the prop reaction, draw the SFD and BMD, and locate the point of contraflexure. | 10M | CO-3 | BL-4 |
| OR |  |  |  |  |
| 6 | A fixed beam AB of span 6 m carries a concentrated load of 90 kN at 2 m from A. Determine the fixed-end moments, support reactions, and draw the SFD and BMD. | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7 | A two-span continuous beam ABC has span AB = 5 m and span BC = 4 m, simply supported at all three supports. AB carries a UDL of 20 kN/m and BC carries a central point load of 40 kN. Using Clapeyron's theorem of three moments, find the support moments and draw the BMD. Take EI constant. | 10M | CO-3 | BL-4 |
| OR |  |  |  |  |
| 8 | A three-span continuous beam ABCD has spans AB = BC = CD = 4 m and is simply supported at all supports. The beam carries a UDL of 15 kN/m on span AB, a central concentrated load of 50 kN on BC, and a UDL of 10 kN/m on CD. Apply the theorem of three moments to find all support moments. Take EI constant. | 10M | CO-3 | BL-3 |
| UNIT-V |  |  |  |  |
| 9 | A simply supported girder of span 20 m is traversed by a uniformly distributed rolling load of 30 kN/m and length 8 m. Calculate (i) the absolute maximum positive shear force, (ii) the absolute maximum negative shear force, and (iii) the absolute maximum bending moment in the girder. | 10M | CO-4 | BL-4 |
| OR |  |  |  |  |
| 10 | Two-wheel loads of 80 kN and 50 kN separated by 3 m cross a simply supported girder of span 12 m from left to right, with the 80 kN load leading. Calculate the maximum shear force and bending moment at a section 4 m from the left support. | 10M | CO-4 | BL-4 |

****