| Subject Code 22PC1ME305 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular and Supplementary Examinations May, 2026

ADDITIVE MANUFACTURING

(ME)

Time: 3 hours                                                                          			 Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | What is the need for Additive Manufacturing in modern product development? List any three key drivers. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Name the materials commonly used in FDM. How does layer thickness affect part quality? | 2M | CO-2 | BL-2 |
|  | How does Binder Jetting differ from Material Jetting in terms of the bonding mechanism? | 2M | CO-3 | BL-2 |
|  | State two advantages of Direct Energy Deposition (DED) processes over SLS. | 2M | CO-4 | BL-1 |
|  | What are the traditional inspection techniques used after AM? When are they preferred? | 2M | CO-5 | BL-1 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Explain the need for AM in modern manufacturing. Describe its applications in any three industries with specific examples. | 5M | CO-1 | BL-2 |
|  | - Discuss the materials used for powder-based AM processes (metals, polymers, ceramics). What are the key material properties required? | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2 | Evaluate the impact of Additive Manufacturing on product development with respect to design freedom, lead time, cost, and material utilization with example. | 10M | CO-1 | BL-5 |
| UNIT-II |  |  |  |  |
| 3 | - With a neat sketch, explain the working principle and process of Fused Deposition Modeling (FDM). State its advantages. | 5M | CO-2 | BL-2 |
|  | - Describe the Laminated Object Manufacturing (LOM) process. List its advantages and limitations. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 4 | Compare Fused Deposition Modeling and Laminated Object Manufacturing in terms of working principle, materials used, accuracy, surface finish, and applications. | 10M | CO-2 | BL-4 |
| UNIT-III |  |  |  |  |
| 5 | - With a neat sketch, explain the SLA process. Discuss the role of photopolymer resin formulation on part accuracy and mechanical properties. | 5M | CO-3 | BL-3 |
|  | - Explain 3D Jetting for multi-material printing. What advantages does it offer over SLA for biomedical component fabrication? | 5M | CO-3 | BL-4 |
| OR |  |  |  |  |
| 6 | Analyze the liquid-based and jetting-based AM processes (SLA, Material Jetting, Binder Jetting) for precision industrial applications. Discuss process selection criteria. | 10M | CO-3 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7 | - Explain the Selective Laser Sintering process with a neat diagram. Mention its advantages and typical applications. | 5M | CO-4 | BL-2 |
|  | - Describe the Laser Engineered Net Shaping process and state its engineering applications. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 8 | Discuss the indirect methods of Rapid Tooling: Investment Cast Tooling, 3D Keltool, and Spray Metal Tooling. Explain their process and suitability. | 10M | CO- | BL-3 |
| UNIT-V |  |  |  |  |
| 9 | - Explain the flatness and warp check inspection technique. Why is warp control critical in AM? | 5M | CO-5 | BL-2 |
|  | - Describe the CAD data formats used in AM (STL, AMF, 3MF). Discuss data loss issues and repair strategies. | 5M | CO-5 | BL-3 |
| OR |  |  |  |  |
| 10 | Evaluate the effectiveness of traditional versus non-destructive inspection methods for AM parts. And also Include flatness check, X-ray CT, and ultrasonic testing in your discussion. | 10M | CO-5 | BL-5 |

****