| Subject Code 22PE1CE306 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular and Supplementary Examinations, May 2026

ADVANCED STRUCTURAL ANALYSIS

(CE)

Time: 3 hours                                                                        				Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A					5X2=10M

|  | Define slope. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Define stiffness factor. | 2M | CO-1 | BL-2 |
|  | Define rotation factor. | 2M | CO-1 | BL-2 |
|  | Define kinematic indeterminacy. | 2M | CO-3 | BL-2 |
|  | List the factors on which the flexibility method depends? | 2M | CO-4 | BL-2 |

PART- B				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | A continuous beam ABC is fixed at A and simply supported at B and C. The span AB is 6m and carries a uniformly distributed load of 10 kN/m. The span BC is 4m and carries uniformly distributed load of 30 kN/m. Determine the fixed end moments by Slope Deflection method. | 10M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2 | A Continuous beam is fixed at A and is supported over rollers at B and C. AB=BC=10M.The beam carries a uniformly distributed load of 20kN/m over AB and a point load of 180kN at a distance of 4M from B on span BC.B has an settlement of 20mm.E= 2.1 x105 N/mm2 ,I= 3 x 109 mm 4 . Analyse the beam by slope deflection method. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | A simply supported beam ABC is continuous over two spans AB and BC of 8m and 6m respectively. Span AB is carrying a uniformly distributed load of 3kN/m and span BC carries point load of 4kN at midpoint of BC. Find the support moment at B if EI of the beam is constant. Use moment distribution method. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4 | Analyze the following frame by Moment Distribution method. | 10M | CO-2 | BL-4 |
| UNIT-III |  |  |  |  |
| 5 | A beam ABC 8m long is fixed at A and simply supported at B with an overhang BC 2m long. The beam carries a uniformly distributed load of 12kN/m on AB and a point load of 12kN at C. Find the support moments and the support reaction. Use Kani’s method. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 6 | Analyze the following frame by Kani’s Method. | 10M | CO-2 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7 | Analyze the beam by stiffness method. | 10M | CO-3 | BL-4 |
| OR |  |  |  |  |
| 8 | Analyse the continuous beam shown in figure by the stiffness method and draw the bending moment diagram. | 10M | CO-3 | BL-4 |
| UNIT-V |  |  |  |  |
| 9 | Analyze the continuous beam shown in figure below by flexibility method. | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 10 | Analyze the continuous beam shown in figure below by flexibility method. | 10M | CO-4 | BL-3 |

****