| Subject Code 22PC1CE304 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular and Supplementary Examinations, May 2026

DESIGN OF STEEL STRUCTURES

(CE)

Time: 3 hours                                                                        			 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

Use of IS 800:2007 & IS Handbook No.1 Charts are permitted.

Assume suitable data if necessary.

PART–A 				             5X2=10M

|  | What is Limit State Design. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | How does ductility affect the strength of a tension member? | 2M | CO-2 | BL-2 |
|  | Define effective slenderness ratio of a built-up column. | 2M | CO-2 | BL-2 |
|  | Write about plastic moment | 2M | CO-3 | BL-1 |
|  | Why are intermediate stiffeners required for plate girders? | 2M | CO-4 | BL-2 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Discuss the important factors that influence the mechanical properties of steel. | 5M | CO-1 | BL-2 |
|  | - Explain briefly about different types of bolted joints and their failures. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2 | Two 10 mm thick plates are connected by a single bolted lap joint with 25 mm dia. bolts. Pitch of the bolts is 60 mm. Calculate the efficiency of joint. Take fu of the plates 410 MPa and 4.6 grade bolts (fub = 400 MPa) | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | - Explain the concept of effective length in compression members. | 5M | CO-2 | BL-2 |
|  | - Derive Euler’s buckling formula. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4 | A column 6 m long is to support a factored load of 4500 kN. The ends of the column are effectively held in position and direction at both ends. Design the column using beam sections and 20 mm thick cover plates. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5 | - Explain the importance of built-up sections. | 5M | CO-2 | BL-2 |
|  | - Compare and contrast laced and battened columns. | 5M | CO-2 | BL-4 |
| OR |  |  |  |  |
| 6 | Design a column with single lacing system to carry a factored axial load of 1500 kN. The effective length of column is 5 m. Use two channels placed toe to toe. | 10M | CO-2 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7 | - Distinguish between static and kinematic theorem in plastic analysis. | 5M | CO-3 | BL-4 |
|  | - Explain about the different types of mechanisms in plastic analysis. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 8 | A simply supported beam of span 6 m (effective) is laterally supported throughout. It carries a total uniformly distributed load of 40 kN including its self-weight. Design the beam and perform the checks for shear, deflection, and web bearing. | 10M | CO-3 | BL-3 |
| UNIT-V |  |  |  |  |
| 9 | - What are the design considerations for flange splices and web splices in plate girders? | 5M | CO-4 | BL-1 |
|  | b) Write the steps involved in the design of plate girder. | 5M | CO-4 | BL-1 |
| OR |  |  |  |  |
| 10 | A welded plate girder has the following elements; Flange plates – 450 mm × 16 mm one plate for each flange, Web – 2500 mm × 10 mm; determine the sectional properties and moment of resistance of the plate girder. | 10M | CO-4 | BL-3 |

****