| Subject Code 22PC1CE306 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular and Supplementary Examinations, May 2026

ENVIRONMENTAL ENGINEERING

(CE)

Time: 3 hours                                                                        			 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | Define design period. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What is super-chlorination? | 2M | CO-2 | BL-1 |
|  | Define sanitation. | 2M | CO-3 | BL-1 |
|  | Define secondary treatment of sewage. | 2M | CO-4 | BL-1 |
|  | List the common operational problems encountered in trickling filters. | 2M | CO-4 | BL-1 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Explain the objectives of a water supply system. | 5M | CO-1 | BL-2 |
|  | - Explain various types of water demands with their typical values. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2 | The population of a town in three consecutive decades was 20,000, 26,000, and 32,000. Estimate the population after 20 years using: (i) Arithmetic increase method (ii) Geometric increase method (iii) Incremental increase method. Also explain the significance and applicability of each method. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | - Explain with sketches the process of plain sedimentation. | 5M | CO-2 | BL-2 |
|  | - Design a rectangular sedimentation tank for a population of 40,000 with: - Water demand = 135 LPCD - Detention time = 6 hours - Depth = 3 m - Assume suitable L/B ratio | 5M | CO-2 | BL-4 |
| OR |  |  |  |  |
| 4 | - A town has a population of 80,000 with a per capita demand of 150 LPCD. Calculate the quantity of chlorine required per day if a chlorine dose of 2 mg/L is applied. | 5M | CO-2 | BL-3 |
|  | - Discuss different forms of chlorine used in water treatment. | 5M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |
| 5 | - Explain conservancy and water carriage systems with merits and demerits. | 5M | CO-3 | BL-2 |
|  | - Explain the process of determination of chloride concentration of a water sample. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6 | Describe the various shapes of sewers commonly adopted in sewerage systems with neat sketches. Explain the advantages and suitability of each shape | 10M | CO-3 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7 | - Describe the construction and working of a skimming tank. | 5M | CO-3 | BL-2 |
|  | - A septic tank is to be designed for a community of 200 people with: Sewage contribution = 120 LPCD Detention period = 24 hours Calculate the capacity of the septic tank. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 8 | A sewage treatment plant receives 10 MLD of sewage having an influent Biochemical Oxygen Demand (BOD) of 200 mg/L. The treatment process employed achieves a BOD removal efficiency of 90%. - Calculate the BOD concentration of the treated effluent. - Determine the daily BOD load removed (in kg/day) by the treatment plant. - Comment on whether the treated effluent is suitable for discharge into inland surface waters as per standard discharge norms. | 10M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |
| 9 | - Explain the principle and working of oxidation ponds. Discuss their suitability for wastewater treatment in tropical regions. | 5M | CO-4 | BL-2 |
|  | - Write a note on zones of pollution in rivers due to sewage discharge. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 10 | Explain sludge management in sewage treatment plants. Describe in detail the processes of sludge thickening and sludge digestion with neat sketches. | 10M | CO-4 | BL-2 |

****