| Subject Code 22PC1AE306 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular and Supplementary Examinations, May 2026

Electric and Hybrid Vehicles

(AE)

Time: 3 hours                                                                          			 Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | Compare EV and HEV. | 2M | CO1 | L2 |
| --- | --- | --- | --- | --- |
|  | Define back EMF in a DC motor. | 2M | CO2 | L1 |
|  | List the power electronic control devices used for motor drive. | 2M | CO3 | L1 |
|  | Sketch the different battery pack technologies with respect to cells arrangement. | 2M | CO4 | L2 |
|  | Calculate the average power required for the accelerating the vehicle with mass of 1064 kg at 86.5 km/hr. in 15 seconds. | 2M | CO5 | L3 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Describe any two types of EV powertrain configurations with neat sketches. | 5M | CO1 | L2 |
|  | - Interpret the various operating modes for the below powertrain with a neat power flow diagram. | 5M | CO1 | L3 |
| OR |  |  |  |  |
| 2 | Examine the various operating modes for the below series–parallel drivetrain that connects an engine and a motor/generator together using planetary gear assembly. Where, engine and motor/generator are connected to the yoke and sun gear, respectively. The ring gear of the planetary gear is connected to the drive wheels through gears of Z1, Z2, Z4, Z5, and a differential. A traction motor is connected to the driven wheels through gears of Z3, Z2, Z4, Z5, and the differential, which couples the output torques of the ring gear and the traction motor together. One clutch and two locks are used. | 10M | CO1 | L4 |
| UNIT-II |  |  |  |  |
| 3 | - Explain the construction and working of PMSM motor with a neat sketch | 5M | CO2 | L2 |
|  | - State the advantages and disadvantages of SRM motors. | 5M | CO2 | L2 |
| OR |  |  |  |  |
| 4 | Explain the construction, equivalent circuit, working principle of BLDC motor with a neat sketch. | 10M | CO2 | L3 |
| UNIT-III |  |  |  |  |
| 5 | - Explain the speed control method of BLDC motor with a neat sketch. | 5M | CO3 | L2 |
|  | - Explain the filed oriented control method of induction motor with a neat sketch. | 5M | CO3 | L2 |
| OR |  |  |  |  |
| 6 | Explain the equivalent circuit and working principle of multi-quadrant control of chopper-fed brushed DC motor drive with a neat sketch. | 10M | CO3 | L2 |
| UNIT-IV |  |  |  |  |
| 7 | - Explain the construction and working of Li-ion battery with a neat sketch. | 5M | CO4 | L2 |
|  | - Explain the construction and working of sodium metal-chloride battery with a neat sketch. | 5M | CO4 | L2 |
| OR |  |  |  |  |
| 8 | The Ni-MH traction battery of an automotive manufacturer model has the following specifications of 168 cells (28 modules), 201.6 V nominal voltage of battery pack, 6.5 Ah nominal capacity (per cell) and nominal Voltage: 1.2V (per cell). Examine the following - A battery can be discharged at a maximum rate of 90 A, and only 50% of its capacity is allowed to be discharged. Determine the time in seconds for which the battery can be used when it is fully charged. - A battery can be charged at a maximum rate of 80 A. The present state of charge (SOC) is 50%. Determine the time required to increase the SOC to 80%. - A battery has an internal resistance of 0.14 Ω during charging and 0.9 Ω during discharging. Determine the efficiency at maximum charge and discharge condition. - A battery has a leakage current of 15 mA. Determine the number of days required for the battery to self-discharge from 80% SOC to 40% SOC. - The battery operates at maximum charging and discharging currents. Determine the voltage drop caused by the internal resistance during charging and discharging. | 10M | CO4 | L4 |
| UNIT-V |  |  |  |  |
| 9 | - Explain the functions of BMS with a block diagram. | 5M | CO5 | L2 |
|  | - Find the approximate rating of an EV powertrain with a vehicle weight of 1767 kg by computing the forces needed to accelerate at 3.87 m/s2. Assuming that aerodynamic, rolling, and hill-climbing force counts for an extra 13% of the needed acceleration force. Also, determine the average and peak powers needed to accelerate the vehicle from 0 to 87.5 km/hr. | 5M | CO5 | L3 |
| OR |  |  |  |  |
| 10 | For a vehicle with mass =1200 kg, frontal area (Af)=3.2m2, drag coefficient (Cd)=0.2, Coefficient of rolling resistance (cr)=0.012, a vehicle speed V=120 km/hr. and with an acceleration a=0.027g. Calculate the traction torque required at the wheels and the corresponding wheel rotational speed (rpm). Assume the radius of tire is 0.317m, density of air is 1.2 kg/m3. | 10M | CO5 | L3 |

****