| uSubject Code 22PC1EE304 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular and Supplementary Examinations May, 2026

ELECTRICAL MEASUREMENTS AND INSTRUMENTATION

(EEE)

Time: 3 hours                                                                        				Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A					5X2=10M

|  | Define Dead time and dead zone. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What is the equation for deflection torque in Dynamometer type instruments and mention the terms involved in it. | 2M | CO-1 | BL-2 |
|  | Mention the differences between V-A and A-V method. | 2M | CO-2 | BL-1 |
|  | What do you mean by standardization of a DC potentiometer? | 2M | CO-3 | BL-2 |
|  | Mention the working principle of a thermistor and also its applications. | 2M | CO-4 | BL-1 |

PART- B				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - What are the different types of errors that are present in the measuring instruments? Explain them in detail. | 5M | CO-1 | BL-2 |
|  | - How is the current range of a PMMC instrument extended with the help of shunts? And list the general requirements of shunts. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2 | - Classify and explain different types of Moving Iron (MI) instruments. | 5M | CO-1 | BL-2 |
|  | - Design a multi range ammeter with ranges of 1A and 5A employing individual shunts in each case. A PMMC meter with an internal resistance of 730 Ω and a full scale current of 5mA is available. | 5M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | - What are the design components of digital energy meter and explain its operation with a circuit diagram. | 6M | CO-1 | BL-2 |
|  | - Write short notes on Automatic Meter Reading (AMR). | 4M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 4 | Discuss the features of dynamometer type ammeter, voltmeter and wattmeter (both Low power factor and Unity power factor) in detail. | 10M | CO-1 | BL-4 |
| UNIT-III |  |  |  |  |
| 5 | - Explain how insulation resistance of a cable can be measured with the help of loss of charge method. | 4M | CO-2 | BL-3 |
|  | - A sample of Bakelite was tested by the Schering bridge method at 33 KV, 50 Hz. Balance was obtained with a standard condenser of 116 PF capacitance, a condenser of capacitance 0.8 μF in parallel with a non reactive resistor of 419 Ω and a non reactive resistor of 220 Ω. Determine the equivalent series resistance and the power factor of then specimen. | 6M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 6 | - Explain the functioning and merits of DeSauty’s bridge. | 4M | CO-2 | BL-2 |
|  | - Explain how the inductance can be measured by using Maxwell bridge with a neat diagram. | 6M | CO-2 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7 | - Describe the operation of a potential transformer by using a simple phasor diagram. | 5M | CO-3 | BL-2 |
|  | - A 100/5A, 50 Hz current transformer has a bar primary and a rated secondary burden of 12.5VA. The secondary winding has 196 turns and a leakage inductance of 0.96mH. With a purely resistive burden at rated full load, the magnetizing mmf is 16AT and the loss excitation required 12A. Find ratio and phase angle errors. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 8 | - Draw the circuit diagram of Crompton’s potentiometer and explain its working. Describe the steps used when measuring an unknown resistance. | 5M | CO-3 | BL-4 |
|  | - Explain in detail how power can be measured using instrument transformers. | 5M | CO-3 | BL-4 |
| UNIT-V |  |  |  |  |
| 9 | - Describe the principle of operation of LVDT. | 5M | CO-4 | BL-2 |
|  | - What is the operating principle of a Transducer, and how are Transducers categorized? | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 10 | - Explain the functioning of a successive approximation type digital voltmeter. | 5M | CO-4 | BL-2 |
|  | - Write short notes on Harmonic distortion analyser. | 5M | CO-4 | BL-2 |

****