| Subject Code 22PE1EE304 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular and Supplementary Examinations, May 2026

Energy Storage Systems

(EEE)

Time: 3 hours                                                                        				Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A					5X2=10M

|  | Define energy storage requirements and list any two domains of application. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | List any two battery modelling methods used for Lithium-ion batteries. | 2M | CO-2 | BL-1 |
|  | Define Single Cell SOC and SOC of series batteries. | 2M | CO-3 | BL-2 |
|  | Define charging efficiency and charging time. | 2M | CO-4 | BL-1 |
|  | State two functions of logic and safety control in BMS. | 2M | CO-5 | BL-1 |

PART- B				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Analyze the storage requirements and storage mechanisms of energy storage systems for different application domains. | 5M | CO-1 | BL-4 |
|  | - An isolated house consumes 7 kWh/day. Battery voltage = 72 V, DoD = 80%, efficiency = 90%. Calculate required battery energy and capacity in Ah. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2 | Discuss terminology and definitions of battery characteristics including energy density, power density, efficiency and states of batteries. | 10M | CO-1 | BL-4 |
| UNIT-II |  |  |  |  |
| 3 | - Explain the reaction mechanism of a Lithium-Ion battery during charging and discharging. | 5M | CO-2 | BL-2 |
|  | - Describe the Equivalent Circuit Model of a Li-ion battery. Briefly explain the Thevenin model | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 4 | Develop the Thevenin model of a series battery pack and explain parameter identification principles. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5 | - Explain the Resistance Method for SOC estimation and discuss the factors affecting its accuracy. | 5M | CO-3 | BL-3 |
|  | - Describe Ampere-hour counting method with limitations. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6 | Compare Kalman Filter, Neural Network, and Adaptive Neuro-Fuzzy Inference System (ANFIS) methods for SOC estimation. Justify their suitability for electric vehicle applications. | 10M | CO-3 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7 | - Explain different charging modes of Lithium-Ion batteries and mention the demerits of present charging methodologies. | 5M | CO-4 | BL-2 |
|  | - Define charging efficiency and charging time. Explain key indicators used to measure charging characteristics. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 8 | Analyze improvements of the CCCV charging method and their impact on battery performance. | 10M | CO-4 | BL-4 |
| UNIT-V |  |  |  |  |
| 9 | - Explain the main functions of a Battery Management System (BMS) in a Li-ion battery pack. | 5M | CO-5 | BL-2 |
|  | - Describe the architecture of a BMS and explain the role of cell voltage, current, and temperature measurement modules. | 5M | CO-5 | BL-2 |
| OR |  |  |  |  |
| 10 | Discuss equalization management circuits and CAN communication in BMS. | 10M | CO-5 | BL-4 |

****