| Subject Code 22OE1EE302 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular and Supplementary Examinations, May 2026

RENEWABLE ENERGY TECHNOLOGIES

(Open Elective)

Time: 3 hours                                                                        			 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | Define efficiency of solar cell? | 2M | CO-1 | BL-3 |
| --- | --- | --- | --- | --- |
|  | List out the few important energy storage methods. | 2M | CO-3 | BL-2 |
|  | Which types of energy storage systems are most suitable for peak shaving in the electric grid? | 2M | CO-3 | BL-2 |
|  | A solar cell has a voltage and current rating of 0.6V and 0.2A. It is desired that 50 cells are to be connected to form a solar module. Determine and compare the voltage and current ratings of the modules for following cases 1. All the 50 cells are connected in series 2. All the 50 cells are connected in parallel | 2M | CO-1 | BL-2 |
|  | What happens to the I-V characteristics of a solar cell when the operating temperature increases. | 2M | CO-2 | BL-1 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Enumerate the mathematical modelling of solar PV cell. | 5M | CO-1 | BL-2 |
|  | - Examine the perturb and observe mppt algorithm for various power and voltage case studies. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2 | Draw the various characteristics of solar cell and give the relation between output power of solar cell with change in temperature and irradiance. | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3 | - What are the different Measuring Parameters of the PV module? Give a brief idea about this. | 5M | CO-3 | BL-2 |
|  | - Discuss all the factors affecting electricity generated by a solar PV module. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 4 | Discuss about - Fixed PV systems - Single-axis tracking PV systems - Dual-axis tracking PV systems. | 10M | CO-3 | BL-2 |
| UNIT-III |  |  |  |  |
| 5 | - What are the key factors when selecting an inverter for a grid-tied PV system? | 5M | CO-3 | BL-2 |
|  | - Why do variable-speed wind turbines use an AC-DC-AC back-to-back converter? | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6 | What is the primary role of a DC-DC converter in a PV system? It acts as an interface between the PV array and the inverter, primarily to perform Maximum Power Point Tracking (MPPT) | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7 | - What is the trade-off between the step size of an MPPT algorithm and its tracking speed during rapid changes in solar irradiance or wind gusts? | 5M | CO-1 | BL-1 |
|  | - How do errors in current and voltage sensor measurements (noise and offset) affect the stability of the MPPT control loop? | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 8 | Why is MPPT often described as an "impedance matching" problem, and how does a DC-DC converter physically achieve this match between the source and the load? | 10M | CO-1 | BL-3 |
| UNIT-V |  |  |  |  |
| 9 | - What is the specific role of the electrolyte in a fuel cell, and why must it be an electronic insulator but an ionic conductor? | 5M | CO-2 | BL-1 |
|  | - What is "methanol crossover," and how does it negatively impact the voltage and efficiency of a DMFC? | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 10 | Compare the operating temperatures and typical applications of PEMFC (Proton Exchange Membrane) versus SOFC (Solid Oxide Fuel Cells). Why are SOFCs better suited for stationary Combined Heat and Power (CHP) systems? | 10M | CO-2 | BL-2 |

****