| Subject Code R22PC1CE305 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular and Supplementary Examinations May, 2026

SOIL MECHANICS
(CE)

Time: 3 hours                                                                                 			Max. Marks: 60

Answer ALL Questions from PART-A

Answer ONE question from each unit of PART-B

PART-A					5X2=10M

|  | Derive the relation between void ratio and porosity. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | A soil sample has LL = 45 and PI = 20. Plot its position on the plasticity chart and classify it. | 2M | CO-2 | BL-1 |
|  | Explain why seepage velocity is always greater than discharge velocity. | 2M | CO-3 | BL-2 |
|  | Define compression index and explain its significance in consolidation analysis. | 2M | CO-4 | BL-1 |
|  | A soil has specific gravity G=2.65and void ratio e=0.70. Calculate the critical hydraulic gradient for quicksand condition. | 2M | CO-3 | BL-3 |

PART-B				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | - Differentiate between specific gravity determination using pycnometer and density bottle. | 5 M | CO-1 | BL-2 |
|  | - A soil sample collected below the water table has a wet density of 1.95 g/cc and dry density of 1.60 g/cc. If G=2.65, calculate porosity, void ratio, and degree of saturation. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
|  | - A hole of volume 500 cm³ is excavated in a soil sample. The weight of soil removed is 950 g. If the dry density of sand used for replacement is 1.60 g/cc, calculate the in-situ density of soil. | 6M | CO-1 | BL-2 |
|  | - A soil embankment requires compaction to a bulk density of 20 kN/m³ at 15% moisture content. The borrow pit soil has a bulk density of 21 kN/m³ and water content of 10%. Determine the excavation volume needed for 1 m³ of embankment. | 4M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
|  | Explain the basis of classification between coarse-grained and fine-grained soils in ISSCS. | 10M | CO-2 | BL-2 |
| OR |  |  |  |  |
|  | - Explain why Atterberg limits are essential in soil classification and engineering design. | 6M | CO-2 | BL-2 |
|  | - A soil pat weighs 0.78 N when oven-dried and displaces 4.75 N of mercury. Using G = 2.72 and Gm = 13.6, outline the steps to compute shrinkage limit. | 4M | CO-2 | BL-3 |
| UNIT- III |  |  |  |  |
|  | - State Darcy’s law and explain how it is used to determine the coefficient of permeability (k). | 5M | CO-3 | BL-3 |
|  | - A soil sample of 6 cm height and 50 cm² area allows 500 m³ of water to flow in 15 minutes under a head of 45 cm. Outline the steps to calculate the coefficient of permeability (k). | 5M | CO-3 | BL-4 |
| OR |  |  |  |  |
|  | - Explain how the average coefficient of permeability is determined in the horizontal direction for a stratified soil deposit. | 5M | CO-3 | BL-3 |
|  | - A dam is constructed with upstream head = 16 m and downstream head = 2 m. The flow net gives Nf = 9 and Nd = 16. Outline the steps to calculate seepage loss through the subsoil. | 5M | CO-3 | BL-3 |
| UNIT- IV |  |  |  |  |
|  | - Describe the role of the Proctor test in establishing compaction standards for field work? | 5M | CO-4 | BL-2 |
|  | - List and briefly describe the different methods of compaction used in the field. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
|  | - Discuss the role of Darcy’s law in Terzaghi’s consolidation theory. | 4M | CO-4 | BL-1 |
|  | - A clay layer of 3 m thickness has Cv = 0.025 cm²/min and final settlement = 8 cm. Outline the steps to calculate the time required for 80% consolidation. | 6M | CO-4 | BL-3 |
| UNIT- V |  |  |  |  |
|  | - Define total vertical pressure, pore water pressure, and effective stress in soil mechanics. | 5M | CO-3 | BL-3 |
|  | - A 6 m thick clay layer lies beneath a 4 m thick sand deposit. The water table is at the ground surface. The saturated unit weight of sand is 20 kN/m³ and that of clay is 18 kN/m³. Compute the total stress, pore water pressure and effective stress at the mid-height of the clay layer. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
|  | - Discuss the advantages and limitations of the vane shear test compared to other shear strength tests. | 5M | CO-4 | BL-2 |
|  | - A triaxial compression test was conducted on a cylindrical soil specimen of diameter 38 mm and height 76 mm. The specimen was subjected to a cell pressure of 200 kN/m². At failure, the deviator stress recorded was 300 kN/m². Assume the soil is cohesionless (c=0). a) Calculate the principal stresses at failure. b) Plot the Mohr’s circle of stress at failure. c) Determine the angle of internal friction (ϕ) of the soil. | 5M | CO-4 | BL-3 |

***