| Subject Code 22PE1ME410 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year II Semester Regular Examinations, May 2026

METAL CASTING TECHNOLOGY

(ME)

Time: 3 hours                                                                          		 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | What are the limitations of green sand moulds? | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Why is shrinkage allowance not required for grey iron castings? | 2M | CO-2 | BL-2 |
|  | For what applications rotary furnace are well suited? | 2M | CO-3 | BL-2 |
|  | Explain the term fettling. | 2M | CO-4 | BL-2 |
|  | What types of defects are detected by liquid penetrant inspection? | 2M | CO-5 | BL-2 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - What are the types of sand used in casting, and define the quality indicators for the sand characteristics | 5M | CO-1 | BL-2 |
|  | - What are the types of bonding materials which are added to mold sand? | 5M | CO-1 | BL-1 |
| OR |  |  |  |  |
| 2 | Compare vertical and horizontal true centrifugal casting processes with neat sketches. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | - Differentiate between pressurized and unpressurised gating systems with reference to the applications. | 5M | CO-2 | BL-3 |
|  | - Describe how runner extension is helpful for good casting quality. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 4 | Describe the types of risers and their uses with suitable sketches. What are the essential conditions that are to be kept in mind while designing risers? | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5 | - Explain the construction and working of a direct-arc electric furnace. For what applications is it most suitable? | 5M | CO-3 | BL-2 |
|  | - Describe the operation of an electric induction furnace. How does frequency of electric supply affect its operation? Explain its process characteristics. Give its advantages and limitations. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6 | What are the parameters affecting the fluidity of the molten metal during pouring, and how to control these parameters? | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7 | - What are the various techniques used in repair of castings? Discuss with examples. | 5M | CO-4 | BL-3 |
|  | - What operations are involved in a fettling shop? Explain each one briefly. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 8 | What are the possible casting defects that may be caused by the improper gating sys tem design? State at least four defects. | 10M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |
| 9 | - Warpage is a common phenomenon in steel castings. Explain with illustrations, why is it caused and how it can be minimized. | 5M | CO-5 | BL-4 |
|  | - Explain the purpose of dye penetrant testing used for non-destructive inspection of castings. | 5M | CO-5 | BL-2 |
| OR |  |  |  |  |
| 10 | Compare magnetic particle inspection and radiographic inspection in terms of the process and application. | 10M | CO-5 | BL-4 |

****