| Subject Code 22PE1EC411 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year II Semester Regular Examinations, May 2026

SATELLITE COMMUNICATIONS

(ECE)

Time: 3 hours                                                                        			 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | What is orbital period? Write the expression relating orbital period and orbital radius. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Define transponder and mention its role in the communication subsystem of a satellite. | 2M | CO-2 | BL-1 |
|  | Define Frequency Division Multiple Access (FDMA) and state one advantage of using it in satellite communications. | 2M | CO-3 | BL-1 |
|  | What is meant by uplink and downlink in satellite communication? | 2M | CO-4 | BL-1 |
|  | What is the basic principle used in GPS for position determination? | 2M | CO-5 | BL-1 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Explain Kepler’s laws as applied to satellite motion. | 5M | CO-1 | BL-3 |
|  | b) Explain various applications of satellite communications in broadcasting, navigation, meteorology, and defense. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2 | Explain look angle determination for Earth station antennas. Derive the expressions for elevation angle and azimuth angle and discuss the effect of orbital perturbations on satellite positioning and link performance. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | - Explain the role of command and monitoring subsystem in satellite control and health management. | 5M | CO-2 | BL-2 |
|  | - Write short notes on telemetry and tracking in satellite communications. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 4 | Explain the Attitude and Orbit Control System (AOCS) of a communication satellite. Describe its components, working principles, and role in maintaining satellite orientation and orbital stability. | 10M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |
| 5 | - Explain Demand Assigned Multiple Access (DAMA) and its advantages in satellite communication networks. | 5M | CO-3 | BL-2 |
|  | - Describe the frame structure of a TDMA system and explain the purpose of reference burst. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6 | Explain Code Division Multiple Access (CDMA) in satellite communications. Discuss spread spectrum transmission and reception principles. | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7 | - Explain the tracking systems used in earth stations for satellite communication. | 5M | CO-4 | BL-2 |
|  | - Write short notes on earth station transmitters and receivers used in satellite communication systems. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 8 | Explain the basic transmission theory used in satellite link design. Derive the link equation and discuss the significance of system noise temperature and G/T ratio in determining link performance. | 10M | CO-4 | BL- |
| UNIT-V |  |  |  |  |
| 9 | - Write short notes on GPS receivers and GPS codes (C/A code and P code). | 5M | CO-5 | BL- |
|  | - Describe Differential GPS (DGPS) and its advantages over standard GPS. | 5M | CO-5 | BL- |
| OR |  |  |  |  |
| 10 | Compare Low Earth Orbit (LEO) and Geostationary Earth Orbit (GEO) satellite systems in terms of orbit considerations, coverage, delay, throughput, and system design requirements. | 10M | CO-5 | BL- |

****