| Subject Code 22PC1EC205 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular and Supplementary Examinations, May 2026

Analog Circuits

(ECE)

Time: 3 hours                                                                        			 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | Why is the maximum efficiency of a Series-fed Class A amplifier limited to 25%? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Define Rise Time (tr) in an RC low-pass circuit and state its relationship with the upper 3dB frequency (fH). | 2M | CO-2 | BL-1 |
|  | What is the main advantage of an Inverted R-2R Ladder DAC over a standard R-2R Ladder? | 2M | CO-3 | BL-1 |
|  | Explain the Virtual Ground concept in Op-Amps and its importance in summing amplifier circuits. | 2M | CO-4 | BL-2 |
|  | Define Capture Range and Lock-in Range of a Phase Locked Loop (PLL). | 2M | CO-5 | BL-1 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | Draw the circuit diagram of a Push-Pull Class B Power Amplifier and derive the expression for its efficiency. | 10M | CO-1 | BL-4 |
| OR |  |  |  |  |
| 2 | Define Tuned Amplifiers. Explain the working and frequency response of a Single Tuned Amplifier with a neat circuit diagram. | 10M | CO-1 | BL-1 |
| UNIT-II |  |  |  |  |
| 3 | - Draw the output waveform of an RC high-pass circuit for a square wave input. Derive the expression for the percentage of tilt. | 5M | CO-2 | BL-4 |
|  | - Explain how an RC low-pass circuit acts as an Integrator. What are the necessary conditions for this operation? | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 4 | Draw the circuits for Positive and Negative Shunt Clippers using diodes. Explain their operation with the help of transfer characteristics. | 10M | CO-2 | BL-4 |
| UNIT-III |  |  |  |  |
| 5 | - Define an Ideal Op-Amp. Compare the ideal and practical characteristics of an Op-Amp. | 5M | CO-3 | BL-4 |
|  | - Derive the expression for the closed-loop voltage gain of an Op-Amp in non-inverting mode. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6 | Explain the operation of an R-2R Ladder Type DAC with a neat circuit diagram. Why is it preferred over the Weighted Resistor DAC? | 10M | CO-3 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7 | - Design a Three Op-Amp Instrumentation Amplifier and derive the expression for its overall gain. Mention its advantages. | 5M | CO-4 | BL-3 |
|  | - Explain the operation of a Sample and Hold circuit with a neat circuit diagram and relevant waveforms. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 8 | Design a LPF at a cut-off frequency of 1kHz with a passband gain of 2. And plot the frequency response. | 10M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |
| 9 | - Draw the block diagram of Phase Locked Loop (PLL) and explain the role of each block. | 5M | CO-5 | BL-4 |
|  | - Explain the operation of an astable multivibrator using 555 timer. Derive the expression for its frequency and duty cycle | 5M | CO-5 | BL-3 |
| OR |  |  |  |  |
| 10 | Explain the operation of Monostable Multivibrator using 555 timer. Derive the expression for the output pulse width | 10M | CO-5 | BL-3 |

****