| Subject Code 22PC1EC210 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular and Supplementary Examinations, May 2026

Analog Electronic Circuits

(EEE)

Time: 3 hours                                                                        			 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | State Miller’s theorem. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Compare the performance of large signal amplifiers with reference to different parameters. | 2M | CO-2 | BL-4 |
|  | Explain the condition for which the low pass filter acts as integrator. | 2M | CO-3 | BL-3 |
|  | Define the terms CMRR and slew rate. | 2M | CO-4 | BL-1 |
|  | List the applications of 555 timers. | 2M | CO-5 | BL-1 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | - With the necessary mathematical equations discuss the effect of coupling and emitter by pass capacitors on frequency response. | 5M | CO-1 | BL-3 |
|  | - For the following low-frequency parameters of a typical transistor at room temperature Ic = 10 mA, VCE = 10 V, hie = 500 Ω, hoe = 4 × 10−5 A/V, hfe = 100, hre = 10−4. At the same operating point, unity gain frequency fT = 50 MHz and C π = 3 pF. Compute the values of all the hybrid- π parameters. | 5M | CO-1 | BL-4 |
| OR |  |  |  |  |  |
| 2 | Draw the hybrid-π common emitter transistor model, illustrating the physical significance of each parameter and derive an equation for the beta cut-off frequency. | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |  |
| 3 | - Discuss the procedure for frequency response analysis of multi-stage amplifiers. | 5M | CO-2 | BL-3 |
|  | - Write short notes on class-C power amplifiers. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |  |
| 4 | Explain the functional operation of class-B push pull power amplifier and derive an equation for conversion efficiency for the same circuit. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |  |
| 5 | - Find the response of a high pass RC circuit for an input of step signal. | 5M | CO-3 | BL-2 |
|  | - Give the operational principle and applications of basic positive clamper circuit. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
| 6 | With necessary circuit diagram and waveforms describe the operation of diode clipping at two independent levels. | 10M | CO-3 | BL-2 |
| UNIT-IV |  |  |  |  |  |
| 7 | - Draw and explain the functional block diagram of basic operational amplifier. | 5M | CO-4 | BL-2 |
|  | - Illustrate the DC, AC characteristics and operating modes of a typical operational amplifier. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |  |
| 8 | Derive an equation for ON-time and OFF-time of a square waveform generated using operation amplifier. | 10M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |  |
| 9 | - With necessary circuit diagram discuss the operation of weighted resistor DAC. | 5M | CO-5 | BL-3 |
|  | b) Explain the operational principle of successive approximation type ADC. | 5M | CO-5 | BL-3 |
| OR |  |  |  |  |  |
| 10 | Illustrate the design of astable multi-vibrator using 555 timer. | 10M | CO-5 | BL-3 |

****