| Subject Code 22PC1AE209 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular and Supplementary Examinations, May/June 2026

APPLIED THERMODYNAMICS

(AE)

Time: 3 hours                                                                        			 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | Write the purpose of condenser. | 2M | CO1 | BL1 |
| --- | --- | --- | --- | --- |
|  | Define critical pressure ratio of a steam nozzle and state its significance. | 2M | CO2 | BL1 |
|  | Identify the effect of clearance volume of a reciprocating compressor on its volumetric efficiency. | 2M | CO1 | BL2 |
|  | List out methods used to improve the thermal efficiency of a gas turbine plant. | 2M | CO2 | BL2 |
|  | Define specific humidity and wet-bulb temperature. | 2M | CO3 | BL1 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | Illustrate with a neat sketch the construction and working of a LaMont high pressure boiler. | 10M | CO1 | BL1 |
| OR |  |  |  |  |
| 2 | Explain the construction and working of evaporative condenser with neat sketch. | 10M | CO1 | BL1 |
| UNIT-II |  |  |  |  |
| 3 | Steam having pressure of 10.5 bar and 0.95 dryness is expanded through a convergent and divergent nozzle and the pressure of steam leaving the nozzle is 0.85 bar. Find the velocity at the throat for maximum discharge conditions. Index of expansion may be assumed as 1.135. Also, calculate mass rate of flow of steam through the nozzle. | 10M | CO2 | BL3 |
| OR |  |  |  |  |
| 4 | In a single-stage impulse turbine, the steam jet leaves the nozzles at 200 to the plane of the wheel at a speed of 670m/s and it enters the moving blades at an angle of 350 to the drum axis. The moving blades are symmetrical in shape. Determine the blade velocity and diagram efficiency. | 10M | CO2 | BL3 |
| UNIT-III |  |  |  |  |
| 5 | A single acting air compressor takes in air at 1 bar 27°C and delivers at 1.4 MPa running at 300 rpm and has cylinder dia of 160 mm and stroke 200 mm. clearance volume is 4% of stroke volume. If the pressure and temperature at the end of the suction stroke is 100 kPa, 47°C and compression and expansion takes places by polytropic process (n=1.2), Determine the mass of air delivered per minute, volumetric efficiency and the driving power required if mechanical efficiency is 85%. | 10M | CO1 | BL4 |
| OR |  |  |  |  |
| 6 | Describe the construction and working of axial flow compressor with neat sketch. | 10M | CO1 | BL2 |
| UNIT-IV |  |  |  |  |
| 7 | Air enters the compressor of a cold air-standard Brayton cycle gas turbine power plant with regeneration and reheat at 100 kPa (abs), 300 K, with a mass flow rate of 6 kg/s. The compressor pressure ratio is 10 and the inlet temperature for each turbine stage is 1400 K. The pressure ratios across each turbine stage are equal. The turbine stages and compressor each have isentropic efficiencies of 80% and the regenerator effectiveness is 80%. For a specific heat ratio of 1.4, Calculate: the thermal efficiency of the cycle, the back work ratio, and the net power developed by the cycle. | 10M | CO2 | BL3 |
| OR |  |  |  |  |
| 8 | A rocket moves with a velocity of 10,000 km/hr with an effective exhaust velocity of 1400 m/sec, the propellant flow rate is 5 kg/sec and the propellant mixture has a heating value of 6500 kJ/kg. Determine propulsion efficiency, engine output power, thermal efficiency and overall efficiency. | 10M | CO2 | BL3 |
| UNIT-V |  |  |  |  |
| 9 | A refrigeration machine using R-12 as refrigerant operates between the pressures 2.5 bar and 9 bar. The compression is isentropic and there is no undercooling in the condenser. The vapour is in dry saturated condition at the beginning of the compression Estimate the theoretical coefficient of performance. If the actual coefficient of performance is 0.65 of theoretical value, calculate the net cooling produced per hour. The refrigerant flow is 5 kg per minute. | 10M | CO3 | BL4 |
| OR |  |  |  |  |
| 10 | The amount of air supplied to an air conditioned hall is 300m3/min. The atmospheric conditions are 35°C DBT and 55% RH. The required conditions are 20°C DBT and 60% RH. Find out the sensible heat and latent heat removed from the air per minute. Also find sensible heat factor for the system. | 10M | CO3 | BL4 |

****