| Subject Code 22PC1AE206 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular and Supplementary Examinations, May 2026

AUTOMOTIVE ENGINES

(AE)

Time: 3 hours                                                                        			 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | List the principal types of IC engines with their classification criteria. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | List the significance of the air-fuel ratio in SI engines? | 2M | CO-1 | BL-1 |
|  | State the function of a throttle position sensor (TPS) and explain how it influences fuel injection. | 2M | CO-2 | BL-1 |
|  | Define degree of supercharging. State its effect on engine power output. | 2M | CO-3 | BL-1 |
|  | Define brake thermal efficiency and mechanical efficiency. Write their mathematical expressions. | 2M | CO-4 | BL-1 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Sketch and explain the actual valve timing diagram of a four-stroke SI engine. Identify the reasons for early opening and late closing of inlet and exhaust valves with reference to engine performance. | 5M | CO-1 | BL-2 |
|  | - Describe the construction, material selection, and function of piston rings. Explain the significance of ring gap and ring-to-groove clearance on engine performance. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2 | Illustrate the working of a four-stroke CI engine with a neat P-V diagram. Compare the theoretical Diesel cycle with the actual cycle. | 10M | CO-1 | BL-1 |
| UNIT-II |  |  |  |  |
| 3 | Describe the three stages of combustion in a petrol engine. Explain how engine speed and ignition timing affect the occurrence and intensity of knocking in SI engines. | 10M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 4 | Describe the different types of combustion chambers used in SI engines (Hemispherical, Wedge, and Pentroof types). For each type, explain the flame travel path, thermal loading, and its susceptibility to detonation. | 10M | CO-1 | BL-4 |
| UNIT-III |  |  |  |  |
| 5 | - With neat sketches, explain the construction and working principles of a hot-wire Mass Air Flow (MAF) sensor and a Manifold Absolute Pressure (MAP) sensor. | 5M | CO-2 | BL-2 |
|  | - Compare Throttle Body Injection (TBI) and Multi-Point Fuel Injection (MPFI) systems under the following criteria: (i) spray quality and fuel atomization, (ii) transient response during acceleration, (iii) cylinder-to-cylinder mixture distribution, and (iv) emission levels. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 6 | Explain the working principle of a Gasoline Direct Injection (GDI) system with a neat block diagram. Describe the injection strategies employed during cold start, part load, and full load operating conditions. | 10M | CO-2 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7 | Draw and explain the working of a pressurized water cooling system. Describe the function of the wax-type thermostat in controlling coolant flow and explain its impact on engine warm-up and fuel economy. | 10M | CO-3 | BL-1 |
| OR |  |  |  |  |
| 8 | Briefly discuss the working of a turbocharger system with a neat diagram showing all components. Discuss: (i) the degree of supercharging and its effect on engine output power; (ii) intercooling and its thermodynamic benefits. | 10M | CO-3 | BL-4 |
| UNIT-V |  |  |  |  |
| 9 | - A four-cylinder, four-stroke petrol engine has the following specifications: bore = 80 mm, stroke = 100 mm, speed = 3000 rpm, indicated mean effective pressure = 7.0 bar, frictional power = 4.2 kW. Determine: (i) Indicated power, (ii) Brake power, (iii) Mechanical efficiency, and (iv) BMEP. | 7M | CO-4 | BL-3 |
|  | - Explain the concept and necessity of a Variable Compression Ratio (VCR) engine. | 3M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 10 | Compare the working principles, combustion strategies, and emission characteristics of HCCI (Homogeneous Charge Compression Ignition), stratified charge, and lean burn engines. | 10M | CO-4 | BL-2 |

****