| Subject Code 22PC1EC208 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular and Supplementary Examinations, May/June 2026

COMPUTER ORGANIZATION AND DESIGN

(ECE)

Time: 3 hours                                                                        			 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | Give basic symbols of the register transfer notation in RTL. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Describe character representation method used in computers. | 2M | CO-1 | BL-1 |
|  | What is a microinstruction? | 2M | CO-2 | BL-1 |
|  | Find number of address and data lines required for 2k X 8 memory. | 2M | CO-3 | BL-1 |
|  | Compare program-controlled I/O and interrupt-driven I/O. | 2M | CO-4 | BL-1 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | Explain the functional blocks of a computer system with a neat diagram. Describe the role of CPU, memory, input-output subsystem, and control unit in overall system operation. | 10M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2 | Define addressing mode. Explain generic addressing modes supported by the CPU with an example. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | - Compare fixed-point and floating-point representations. | 5M | CO-1 | BL-2 |
|  | - Represent the given decimal numbers in 5-bits using 2’s complement notation. Perform the specified arithmetic operations and identify whether overflow occurs in each case. i) (+13) + (+7) ii) (+13) - (+7) iii) (-12) + (+9) iv) (-12) - (+9) | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 4 | Describe Booth’s multiplication algorithm with a suitable example. | 10M | CO-1 | BL-3 |
| UNIT-III |  |  |  |  |
| 5 | a) Compare hardwired control and microprogrammed control units in terms of speed, flexibility, and design complexity. | 5M | CO-2 | BL-2 |
|  | b) Describe the general configuration of a microprogrammed control unit with a neat block diagram. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 6 | Explain in detail the process of address sequencing in a microprogrammed control unit with the help of a neat diagram. | 10M | CO-2 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7 | - Analyze how hierarchical memory organization improves system performance and cost efficiency. | 5M | CO-3 | BL-4 |
|  | - Analyze how memory interleaving increases memory access speed. | 5M | CO-3 | BL-4 |
| OR |  |  |  |  |
| 8 | Explain various cache memory mapping techniques with an example. | 10M | CO-3 | BL-3 |
| UNIT-V |  |  |  |  |
| 9 | - Explain about SCSI and USB interfaces. | 4M | CO-4 | BL-2 |
|  | - Explain DMA controller with a neat block diagram. | 6M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 10 | a)Explain the concept of pipelining in processors | 5M | CO-4 | BL-2 |
|  | b) Explain different array processors in detail. | 5M | CO-4 | BL-2 |

****