| Subject Code 22PC1DS202 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular and Supplementary Examinations, May/June 2026

DIGITAL LOGIC DESIGN AND COMPUTER ORGANIZATION

(Common to CS-DS, CS-AIML & AIDS)

Time: 3 hours                                                                        			 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | Convert (101101)2 into decimal. | 2M | CO1 | BL2 |
| --- | --- | --- | --- | --- |
|  | Write the excitation table of JK flip-flop. | 2M | CO2 | BL2 |
|  | Define addressing mode. | 2M | CO3 | BL1 |
|  | Define Hardwired Control Unit. | 2M | CO4 | BL1 |
|  | List different types of I/O transfer techniques. | 2M | CO5 | BL1 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Realize XOR gate using NAND gate. | 5M | CO1 | BL3 |
|  | - Minimize using K-map F(A,B,C)=Σm(1,3,5,7) | 5M | CO1 | BL3 |
| OR |  |  |  |  |
| 2 | a)Express the Boolean expression in Sum of Min terms F=A+BC. | 5M | CO1 | BL2 |
|  | b)Find the complement and dual of (X’.Y)+(X.Y’) | 5M | CO1 | BL2 |
| UNIT-II |  |  |  |  |
| 3 | - Design a 4-to-1 Multiplexer using logic gates. | 5M | CO2 | BL3 |
|  | - Convert JK Flip-Flop into D Flip-Flop. | 5M | CO2 | BL3 |
| OR |  |  |  |  |
| 4 | a)Design a Priority Encoder (4-to-2) with truth table and logic diagram | 5M | CO2 | BL3 |
|  | b)Realize full adder using two half adders. | 5M | CO2 | BL4 |
| UNIT-III |  |  |  |  |
| 5 | - Explain the Instruction Execution Cycle with neat diagram. | 5M | CO3 | BL2 |
|  | - Explain Fixed-point and Floating-point representation. | 5M | CO3 | BL2 |
| OR |  |  |  |  |
| 6 | Perform multiplication using Booth’s Algorithm: (−7)×(5) and clearly explain each step. | 10M | CO3 | BL3 |
| UNIT-IV |  |  |  |  |
| 7 | - Explain the organization of a micro-programmed control unit with diagram. | 5M | CO4 | BL1 |
|  | - Explain different types of pipeline hazards and methods to overcome them. | 5M | CO4 | BL3 |
| OR |  |  |  |  |
| 8 | A 4-stage pipeline has stage delays of 10 ns, 15 ns, 20 ns and 15 ns. Determine clock cycle time, execution time for 12 instructions and calculate speedup over non-pipelined system. | 10M | CO4 | BL4 |
| UNIT-V |  |  |  |  |
| 9 | - Explain the concept of memory hierarchy with diagram. | 5M | CO5 | BL1 |
|  | - Describe DMA transfer technique and its advantages. | 5M | CO5 | BL2 |
| OR |  |  |  |  |
| 10 | Explain the organization of Cache Memory and discuss mapping techniques | 10M | CO5 | BL2 |

****