| Subject Code 22PC1EE203 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular and Supplementary Examinations, May 2026

ELECTRICAL MACHINES-II

(EEE)

Time: 3 hours                                                                        			 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | Describe the relation between torque and rotor power factor. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What are the limitations of speed control by conventional methods in an Induction motor? | 2M | CO-3 | BL-3 |
|  | Draw the equivalent circuit of a single-phase induction motor. | 2M | CO-2 | BL-2 |
|  | What are the advantages and disadvantages of estimating the voltage regulation of an Alternator by EMF method? | 2M | CO-4 | BL-4 |
|  | What happens when excitation of a synchronous generator is increased? | 2M | CO-4 | BL-2 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | Explain briefly the principle of operation of an induction Motor and the development of Induced Torque in an Induction Motor. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 2 | In a 6 pole, 3 phase, 50 Hz Induction motor with star connected Rotor, the rotor resistance per phase is 0.3 Ω, the reactance at standstill is 1.5Ω per phase and the EMF between the slip rings on open circuit is 175 V. Calculate: (i) Slip at a speed of 960RPM (ii) Rotor EMF per phase (iii) Rotor frequency and reactance at speed of 950 RPM | 10M | CO-4 | BL-4 |
| UNIT-II |  |  |  |  |
| 3 | - Describe the construction and working principle of a Doubly Fed Induction Machine (DFIM). | 5M | CO-1 | BL-2 |
|  | - Explain the starting methods of 3 Phase Induction Motor. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 4 | Explain the circle diagram of a 3-phase induction motor and describe the procedure to draw it. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5 | Explain capacitor-start and capacitor-start capacitor-run motors with torque-speed characteristics. | 10M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 6 | Explain why a single-phase induction motor is not self-starting using double field revolving theory. | 10M | CO-3 | BL-1 |
| UNIT-IV |  |  |  |  |
| 7 | - Explain synchronization procedure of alternators with neat diagram. | 5M | CO-2 | BL-2 |
|  | - Explain the excitation control of 3-phase Synchronous Generator against constant load. | 5M | CO-3 | BL-4 |
| OR |  |  |  |  |
| 8 | Explain how EMF method is used to find out voltage regulation of a 3-phase Alternator with derivation. | 10M | CO-3 | BL-2 |
| UNIT-V |  |  |  |  |
| 9 | - Explain the operation of synchronous motor with phasor diagram. | 5M | CO-4 | BL-1 |
|  | - Explain V-curves and inverted V-curves of synchronous motor. | 5M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 10 | - Explain what hunting is in a Synchronous Motor and the methods of its suppression. | 5M | CO-4 | BL-2 |
|  | - Explain the synchronous condenser and mention its applications. | 5M | CO-3 | BL-4 |