| Subject Code 22ES1ME211 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular and Supplementary Examinations, May 2026

Electrical Technology and Mechanical Technology

(CE)

Time: 3 hours                                                                        				Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A				5X2=10M

|  | What do you understand by an Internal Combustion Engine? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | List any two advantages of arc welding. | 2M | CO-2 | BL-1 |
|  | Mention any two uses of conveyors in construction projects. | 2M | CO-2 | BL-2 |
|  | Classify DC Motor. | 2M | CO-3 | BL-2 |
|  | List the various types of indicating instruments. | 2M | CO-4 | BL-1 |

PART- B				5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | - Explain the working principle of a four-stroke petrol engine with a neat sketch. | 5M | CO-1 | BL-2 |
|  | - Compare and contrast between two-stroke and four-stroke I.C. Engines. | 5M | CO-1 | BL-4 |
| OR |  |  |  |  |  |
| 2 | Explain the refrigeration system with I.C. block diagram and describe desirable properties of a good refrigerant. | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |  |
| 3 | - Describe the process used in arc welding with a neat diagram. | 5M | CO-2 | BL-2 |
|  | - Explain gas welding with a neat diagram. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |  |
| 4 | Describe the process of Riveting with a neat diagram and state its applications. | 10M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |  |
| 5 | - Explain the working of a bulldozer and its applications. | 5M | CO-2 | BL-2 |
|  | - Describe screw and roller conveyors. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |  |
| 6 | Discuss different earth moving equipment and explain the working of a concrete mixer. | 10M | CO-2 | BL-2 |
| UNIT-IV |  |  |  |  |  |
| 7 | - Explain the working principle of a single-phase transformer with neat diagrams. | 6M | CO-3 | BL-2 |
|  | - The maximum flux density in the core of a 250/3000V, 50Hz 1-phase transformer is 1.2Wb/m2. If the emf per turn is 8V, determine i) Primary and Secondary turns ii) Area of core. | 4M | CO-3 | BL-3 |
| OR |  |  |  |  |  |
| 8 | - Explain the working principle of a DC generator with neat diagrams. | 6M | CO-3 | BL- 2 |
|  | - A four-pole generator, having wave-wound armature winding has 45 slots, each slot containing 16 conductors. What will be the voltage generated in the machine when driven at 1500 rpm assuming the flux per pole to be 9 mWb? | 4M | CO-3 | BL-3 |
| UNIT-V |  |  |  |  |  |
| 9 | - Explain the principle of operation of an alternator. | 5M | CO-4 | BL-2 |
|  | - Explain the principle of PMMC instrument with neat sketch. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |  |
| 10 | Explain the torque-slip characteristics and applications of an induction motor. | 10M | CO-4 | BL-2 |

****