| Subject Code 22PC1EI203 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular and Supplementary Examinations, May 2026

Electronic Devices and Circuits-II

(EIE)

Time: 3 hours                                                                        			 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | What is the difference b/w linear wave shaping and nonlinear wave shaping? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Why is Darlington pair not suitable for high-frequency applications? | 2M | CO-2 | BL-2 |
|  | What is maximum power dissipation rating? | 2M | CO-3 | BL-1 |
|  | What does “I” region represent in PIN diode? | 2M | CO-4 | BL-1 |
|  | What is the main advantage of TRIAC over SCR? | 2M | CO-4 | BL-2 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | Design a symmetrical square wave whose peak-to-peak amplitude is 2V and whose average voltage is zero is applied to an RC integrating circuit. The time constant equals to the half period of square wave. Find the peak-to-peak output amplitude? |  | 10M | CO-1 | BL-3 |
| OR |  |  |  |  |  |
| 2 | Design the circuits of any two types of shunt clippers and explain their operation with the help of their transfer characteristics. | 10M |  | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |  |
| 3 | Draw the circuit diagram of CC-CC Darlington pair and explain its working. Derive the expressions for overall current gain, input and output impedance. | 10M |  | CO-2 | BL-3 |
| OR |  |  |  |  |  |
| 4 | For the two-stage RC coupled CE amplifier, determine the overall voltage gain, current gain, input and output resistances. Rc1=10KΩ,Rc2=6.8KΩ, R1=R3=100kΩ and R2=R4=22kΩ Assume hie=1.3KΩ, hfe=60, and hoe=20x10-6 A/V |  | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |  |
| 5 | A class B push pull power amplifier drive a load of 16Ω, connected to the secondary of the ideal transformer. The supply voltage is 25 V. If the no.of turns on the primary is 200 and the no.of turns on the secondary is 50, Calculate (a) Efficiency (b) Max. Power dissipation per transistor. | 10M |  | CO-3 | BL-3 |
| OR |  |  |  |  |  |
| 6 | - Draw the circuit and prove that the maximum efficiency of a transformer-coupled power amplifier is 50%. |  | 5M | CO-3 | BL-3 |
|  | - Calculate the harmonic distortion component for an output signal having a fundamental amplitude of 3V and a second harmonic amplitude of 0.25V. |  | 5M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |  |
| 7 | - Explain the construction and working of a Photo Transistor with a neat sketch. Give its applications | 3M |  | CO-4 | BL-2 |
|  | - Explain the working of tunnel diode with atleast five energy level diagrams. Draw the forward and reverse VI characteristics . | 7M |  | CO-4 | BL-2 |
| OR |  |  |  |  |  |
| 8 | Explain the working of UJT Relaxation Oscillator with circuit diagram and waveform. Derive the expression for frequency of a UJT relaxation oscillator in terms of intrinsic stand-off ratio (η), resistance (R), and capacitance (C). |  | 10M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |  |
| 9 | - Explain the construction, operation and characteristics of DIAC with a neat sketch. |  | 3M | CO-4 | BL-2 |
|  | - Explain with neat diagram the class A commutation technique and its waveforms. |  | 7M | CO-4 | BL-2 |
| OR |  |  |  |  |  |
| 10 | Explain the construction, working principle and V–I characteristics of SCR. Discuss its two-transistor analogy. |  | 10M | CO-4 | BL-2 |