| Subject Code 22PC1ME207 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular and Supplementary Examinations, May 2026

Fluid Mechanics and Hydraulic Machines

(ME)

Time: 3 hours                                                                        			 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | Define specific weight, specific volume, and specific gravity of a fluid. State their SI units. | 2M | CO-1,2 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What is a Venturimeter? Write the expression for actual discharge through a Venturimeter. | 2M | CO-1,2 | BL-1 |
|  | Write about impact of jet? | 2M | CO-1,2 | BL-1 |
|  | Define unit speed, unit discharge, and unit power for a hydraulic turbine. | 2M | CO-1,3 | BL-1 |
|  | What is cavitation? State the conditions that lead to cavitation in centrifugal pumps. | 2M | CO-1,3 | BL-1 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | - The stream function for a 2-dimensional flow is given by =2xy.Calculate the velocity at the point P(2,3).Find the velocity function . | 5M | CO-1,2 |  | BL-2 |  |
|  | - A U-tube differential manometer connects two pipes A and B. Pipe A contains oil (SG = 0.9) and pipe B contains water. The centre of pipe A is 2.5 m above pipe B. The manometer fluid is mercury (SG = 13.6). The deflection in the manometer left column is 20 cm. Find the pressure difference between pipes A and B. | 5M | CO-1,2 |  | BL-3 |  |
| OR |  |  |  |  |  |  |
| 2 | Derive the continuity equation in three dimensions for steady compressible flow. Reduce it to the form applicable for steady incompressible flow. A pipe of 30 cm diameter carries a fluid at a mean velocity of 2 m/s. The pipe forks into two pipes of 20 cm and 10 cm diameter. If the velocity in the 20 cm pipe is 2.5 m/s, find the velocity in the 10 cm pipe. | 10M |  | CO-2,3 |  | BL-4 |
| UNIT-II |  |  |  |  |  |  |
| 3 | - Derive an expression for the velocity of flow through a Pitot tube using Bernoulli’s equation. | 5M | CO-2,3 |  | BL-2 |  |
|  | - Water flows through a horizontal pipe of diameter 15 cm at a velocity of 3 m/s. The pipe then reduces to 10 cm diameter. Using Bernoulli’s equation, find the pressure difference between the two sections. Also find the flow rate. (ρ = 1000 kg/m³) | 5M | CO-2,3 |  | BL-2 |  |
| OR |  |  |  |  |  |  |
| 4 | Derive the Darcy-Weisbach equation for friction head loss in a pipe. Water is pumped through a pipe 500 m long and 250 mm in diameter at 0.08 m³/s. Friction factor f = 0.018. Find: (i) head loss due to friction, (ii) power required to drive the pump if overall efficiency is 75%. | 10M |  | CO-3,4 |  | BL-3 |
| UNIT-III |  |  |  |  |  |  |
| 5 | - A jet of water of diameter 6 cm strikes a flat plate normally with a velocity of 20 m/s. The plate moves in the direction of the jet at 8 m/s. Find: (i) force exerted on the plate, (ii) work done per second, and (iii) efficiency of the jet. | 5M | CO-3,4 |  | BL-2 |  |
|  | - Derive the expressions for force exerted by a jet of water on a series of flat vanes mounted on a wheel. Show that maximum efficiency is 50% when the vane speed is half the jet speed. | 5M | CO-3,4 |  | BL-3 |  |
| OR |  |  |  |  |  |  |
| 6 | A jet of water having a velocity of 35 m/s strikes a curved vane moving with a velocity of 15 m/s. The jet makes an angle of 30° with the direction of motion. The vane angle at exit is 15°. Assuming no friction, find: (i) force exerted by the jet tangentially (ii) work done per second per unit weight of water, and (iii) hydraulic efficiency. | 10M |  | CO-3,4 |  | BL-3 |
| UNIT-IV |  |  |  |  |  |  |
| 7 | - Describe the construction and working of a Pelton wheel with a neat sketch. | 5M | CO-2,4 |  | BL-2 |  |
|  | - A Pelton wheel is to be designed for a head of 60 m when running at 200 rpm. The Pelton wheel produces 95.6475 kW shaft power. The speed ratio = 0.45, Cv = 0.98, overall efficiency = 85%. Find the diameter of the jet and the diameter of the wheel. | 5M | CO-3,4 |  | BL-3 |  |
| OR |  |  |  |  |  |  |
| 8 | A Francis turbine has a runner diameter of 1.2 m at inlet and 0.8 m at outlet. The runner speed is 250 rpm. Width at inlet is 0.2 m. The water leaves radially at the outlet. The flow velocity remains constant at 3 m/s. Determine: (i) guide vane angle, (ii) runner vane angle at inlet, (iii) runner vane angle at outlet, (iv) work done per kg of water, and (v) hydraulic efficiency if head is 30 m. | 10M |  | CO-2,4 |  | BL-4 |
| UNIT-V |  |  |  |  |  |  |
| 9 | - With a neat sketch explain the working of a single-acting reciprocating pump. | 5M | CO-2,4 |  | BL-3 |  |
|  | - A single-acting reciprocating pump has a cylinder diameter of 150 mm and a stroke of 250 mm. The pump runs at 60 rpm. Suction head is 5 m and delivery head is 25 m. Determine: (i) theoretical discharge, (ii) theoretical power, and (iii) if the actual discharge is 3.8 litres/s, find the slip and percentage slip. | 5M | CO-3,4 |  | BL-4 |  |
| OR |  |  |  |  |  |  |
| 10 | A centrifugal pump delivers water against a net head of 14.5 m and design speed is 1000 rpm. The vanes are curved back at 30° to the tangent. The impeller diameter is 300 mm and outlet width is 50 mm.Take manometric efficiency is 95%. Determine: (i) flow velocity at outlet, (ii) absolute velocity at outlet, (iii) angle of absolute velocity at outlet, (iv) discharge. | 10M |  | CO-3,4 |  | BL-4 |

****