| Subject Code 22ES1EE103 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular and Supplementary Examinations, May 2026

FUNDAMENTALS OF ELECTRICAL AND ELECTRONICS ENGINEERING

(ME)

Time: 3 hours                                                                        				Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A					5X2=10M

|  | State Kirchoff’s Current Law (KCL) with an example. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Mention the working principle of DC generator. | 2M | CO-3 | BL-2 |
|  | List the different losses in a transformer. | 2M | CO-2 | BL-2 |
|  | Define the term slip. | 2M | CO-3 | BL-1 |
|  | List the applications of SCR? | 2M | CO-4 | BL-1 |

PART- B				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Calculate the equivalent resistance between A&B using series and parallel combination networks. | 5M | CO-1 | BL-3 |
|  | - A single-phase AC circuit which consists of a capacitance of 5µF connected in series with 10 ohms resistor across a 230V, 50Hz supply. Calculate the capacitive reactance, impedance of the circuit and current flowing through the circuit. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2 | Derive the expression for average value and RMS value of a sinewave. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | - Derive the EMF equation of a DC generator. | 5M | CO-3 | BL-2 |
|  | - An 8 pole DC shunt generator with 778 wave connected conductors and running at 600 rpm. Supplies a load of 10 Ω at a terminal voltage of 250 V. The armature resistance is 0.2 Ω and shunt field resistance of 250 Ω. Find the armature current, induced EMF and flux per pole. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 4 | Discuss the armature and field control techniques for the speed control of a dc motor. | 10M | CO-3 | BL-3 |
| UNIT-III |  |  |  |  |
| 5 | - Describe the principle of operation of a single-phase transformer in detail. | 5M | CO-2 | BL-2 |
|  | - With a neat circuit diagram discuss the open circuit (OC) test on a single-phase transformer. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 6 | - Derive the EMF equation of a transformer. | 5M | CO-2 | BL-3 |
|  | - A 10kVA, 2500/250V, 50Hz, single phase transformer gave the following test results: OC Test (on Low Voltage side): 250V, 0.8A, 50W; SC Test (on High Voltage side): 60V, 3A, 45W. Compute the voltage regulation at full load and 0.8p.f. leading. | 5M | CO-2 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7 | Discuss the concept of finding the voltage regulation of alternator using synchronous impedance method. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 8 | Explain the torque – slip characteristics of a three-phase induction motor with neat sketch. | 10M | CO-3 | BL-2 |
| UNIT-V |  |  |  |  |
| 9 | - Discuss the characteristics of Light Emitting Diode (LED). | 5M | CO-4 | BL-2 |
|  | - Explain the operation of AND, OR, NOT and NAND gates with truth table. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 10 | Sketch and explain the I-V characteristics of PN junction diode. | 10M | CO-4 | BL-2 |

****