| Subject Code 22PC1CE206 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular and Supplementary Examinations, May 2026

HYDRAULIC ENGINEERING AND HYDRAULIC MACHINES

(CE)

Time: 3 hours                                                                          			 Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | For a rectangular channel with bed width 4 m, flow depth 1.8 m and Manning’s n = 0.022, calculate the Chezzy’s coefficient C. | 2M | CO-1 | BL-3 |
| --- | --- | --- | --- | --- |
|  | What is meant by Hydraulic Jump? | 2M | CO-1 | BL-1 |
|  | Find force exerted by jet of water of diameter 100 mm on a stationary flat plate, when strikes the plate normally with a velocity of 30 m/s. | 2M | CO-3 | BL-3 |
|  | What is the use of multistage centrifugal pump? | 2M | CO-3 | BL-1 |
|  | Explain dimensional homogeneity with example. | 2M | CO-3 | BL-1 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | Derive Chezy’s equation for mean velocity estimation in Open Channel. | 10M | CO-1 | BL-4 |
| OR |  |  |  |  |  |
| 2 | What do you understand by open channel flow? Differentiate between (i) steady flow and unsteady flow (ii) Uniform flow and non-uniform flow (iii) Critical flow, sub- critical flow, super critical flow and (iv) Laminar and turbulent flow in an open channel. | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |  |
| 3 | Explain the terms mild, critical, steep, horizontal and adverse slopes. | 10M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
| 4 | - If in a Hydraulic Jump occurring in a Rectangular channel, the Froude number before jump is 10 and energy loss in jump 3m, Estimate i) Sequent depth ii) Discharge / unit width iii) Froude number after jump. | 5M | CO-1 | BL-3 |
|  | - Classify Hydraulic Jumps based on Froude number. | 5M | CO-1 | BL-2 |
| UNIT-III |  |  |  |  |  |
| 5 | A jet of water moving at 40 m/s strikes a curved vane which is moving with a velocity of 18 m/s. The jet makes an angle of 300 with direction of motion of vane at inlet and leaves at an angle of 900 to the direction of motion of vane at outlet. i) Daw neat velocity triangles ii) Calculate vane angles so that water enters and leaves vane without shock (Assume u1=u2) | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |  |
| 6 | - Explain three types of similarities of Dimensional Analysis. | 5M | CO-2 | BL-2 |
|  | - In 1 in 40 model of spillway, the velocity and discharge are 2m/s and 3m3/s. Find the corresponding velocity and discharge in prototype. | 5M | CO-2 | BL-3 |
| UNIT-IV |  |  |  |  |  |
| 7 | Design a Francis turbine runner with the following data. Net head = 68 m, Speed = 750 rpm, Output power = 330 kw, Hydraulic efficiency = 94%, Over all efficiency = 85%, Flow ratio = 0.15, Breadth ratio = 0.1, Inner dia. of runner is 0.5 outer dia. Also assume 6% of circumferential area of the runner to be occupied by the thickness of the vanes. Velocity of flow remains constant throughout and flow is radial at exit. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |  |
| 8 | Draw a neat diagram of Pelton turbine and explain its working. | 10M | CO-3 | BL-2 |
| UNIT-V |  |  |  |  |  |
| 9 | The outer diameter of impeller of centrifugal pump is 200 mm and width 50mm. The pump is running at 1000 rpm and is working against total head of 30m. the vane angle at outlet is 450 and manometric efficiency is 90%. Determine i) Velocity of flow at outlet ii) Discharge iii) Draw velocity triangles. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |  |
| 10 | - 3 turbo generators each of capacity of 20,000 kw have been installed at a Hydro Power station. During certain period, the load on the plant varies from 30,000 kw to 50,000 kw. Calculate i) total installed capacity ii) Load factor iii) utilization factor iv) Plant factor. | 5M | CO-4 | BL-3 |
|  | - Explain multi-stage centrifugal pumps using neat diagram. | 5M | CO-3 | BL-2 |

****