| Subject Code 22PC1EE204 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular and Supplementary Examinations, May 2026

POWER SYSTEMS –I

(EEE)

Time: 3 hours                                                        					Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | What is the purpose of Economiser and Air pre-heater in thermal power plant? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Explain skin effect in transmission lines. | 2M | CO-2 | BL-1 |
|  | - Write the coefficient and sign of reflections for voltage and current waves in the line terminated with open circuited end. | 2M | CO-3 | BL-1 |
|  | Define string efficiency. | 2M | CO-2 | BL-1 |
|  | From what point of view are feeders and distributors designed in a distribution system? | 2M | CO-4 | BL-1 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | - Explain the operation of Hydro Electric power plant with neat sketch. | 5M | CO-1 | BL-2 |
|  | - Explain advantages and disadvantages of nuclear power plant. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
| 2 | - Explain the operation of Thermal power plant. | 5M | CO-1 | BL-2 |
|  | - Explain advantages and disadvantages of gas power plant. | 5M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |  |
| 3 | - What is the need for transposition of 3-Ф unsymmetrically spaced transmission lines? | 4M | CO-2 | BL-1 |
|  | - A 3-phase, 50 Hz transmission line is 120 km long and delivers 25 MW at 132 kV to a load at 0.8 pf lagging. Line constants per phase are: R=0.15 Ω/km X=0.3 Ω/km, Y=j3×10-6 mho/km Using Nominal-π method, determine Sending-end voltage, Sending-end current and Voltage regulation. | 6M | CO-2 | BL-4 |
| OR |  |  |  |  |  |
| 4 | Determine Transmission line (ABCD) parameters of long transmission line. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |  |
| 5 | - Explain travelling wave theory in transmission lines. | 5M | CO-3 | BL-3 |
|  | - Explain and draw the voltage and current variations in the line terminated with short circuited end. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |  |
| 6 | - Derive expression for critical disruptive voltage. | 5M | CO-3 | BL-2 |
|  | - A 3-phase, 132 kV, 60 Hz transmission line consists of 1.8 cm radius conductor spaced 1.8 meters apart in equilateral triangular formation. If the temperature is 46ºC and atmospheric pressure is 82 cm, Take mo = 0·85 calculate the corona loss per km of the line (consider dielectric strength, g0=21.21 kV (rms)/cm). | 5M | CO-3 | BL-4 |
| UNIT-IV |  |  |  |  |  |
| 7 | - Derive an expression for Sag when the supports are at equal heights. | 5M | CO-2 | BL-3 |
|  | - An overhead transmission line has a span of 300 m between two supports at equal height. The conductor weighs 1.2 kg/m and the maximum allowable tension is 2500 kg. Find Sag in the conductor and Height of conductor at mid-span if supports are 20 m high. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |  |
| 8 | - Explain different methods to improve the string efficiency of suspension type insulators. | 6M | CO-2 | BL-3 |
|  | - What are the advantages of suspension type insulators over other type. | 4M | CO-2 | BL-2 |
| UNIT-V |  |  |  |  |  |
| 9 | - Classify the Distribution systems based on various aspects (Nature of current, type of construction, scheme of connection). | 5M | CO-4 | BL-2 |
|  | - What are the requirements of Distribution systems? | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |  |
| 10 | - For a simple a.c radial distribution system with two load points, obtain the relevant mathematical expressions and draw the phasor diagram for a case when power factors are referred to receiving and voltage. | 5M | CO-4 | BL-3 |
|  | - A 2-wire DC distributor AB is 800 m long and is fed from end A at 240V. Loads are tapped as follows 40 A at 200 m from A , 60 A at 500 m from A and 30 A at 800 m from A Resistance of each conductor = 0.05 Ω/km Find Voltage at each load point. | 5M | CO-4 | BL-3 |