| Subject Code 22PC1CE204 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular and Supplementary Examinations, May 2026

STRENGTH OF MATERIALS-II

(CE)

Time: 3 hours							   			Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | a) | Define principal stresses and principal planes. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- | --- |
|  | b) | Define close-coiled helical spring. | 2M | CO-2 | BL-1 |
|  | c) | Write Lame’s equations for thick cylinders and explain the terms. | 2M | CO-3 | BL-2 |
|  | d) | State the limitations of Euler’s formula. | 2M | CO-4 | BL-1 |
|  | e) | What is middle-third rule? | 2M | CO-4 | BL-2 |

PART-B				         5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1) | At a point in a strained material the principal tensile stresses across two perpendicular planes are 80 N/mm2 and 40 N/mm2. Determine the normal stress, shear stress and the resultant stress on a plane inclined at 200 with the major principal plane. Also, determine the obliquity. | 10M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2) | A rectangular block of a material is subjected to a tensile stress of 110 N/mm2 on one plane and a tensile stress of 47 N/mm2 on the plane right angles to the former. Each of the above stresses is accompanied by a shear stress of 63 N/mm2. Find the (i) Direction and magnitude of principal stress and (ii) Magnitude of maximum shear stress. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3) | A hollow circular shaft has an external diameter of 120 mm and the internal diameter is three-fourths the external diameter. Find the maximum torque that can be transmitted if the allowable shear stress is 36 MPa. Also calculate the angle of twist per unit length. Take the modulus of rigidity of the shaft material as 85 MPa. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4) | A closely coiled helical spring of round steel wire 10 mm in diameter having 10 complete turns with a mean diameter of 120 mm is subjected to an axial load of 200 N. Determine the (i) Deflection of the spring; (ii) Maximum shear stress in the wire and (iii) Stiffness of the spring. Take C = 8 x 104 N/mm2. | 10M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |
| 5) | A shell 3.25 m long, 1 m in diameter is subjected to an internal pressure of 1 N/mm2. If the thickness of the shell is 10 mm, find the circumferential and longitudinal stresses. Find also the maximum shear stress and the changes in the dimensions of the shell. Take E = 2 X 105 N/mm2 and 1/m = 0.3. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6) | A thick cylinder of internal diameter 200 mm is required to withstand a pressure of 30 N/mm2. If the permissible hoop stress is 120 N/mm2, calculate the external diameter and minimum value of hoop stress in the cylinder. Sketch the radial pressure and hoop stress distribution across the section. | 10M | CO-3 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7) | Derive the Euler’s formula for finding the critical load in a long column of length ‘L’ hinged at both ends, having flexural rigidity EI. | 10M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 8) | A hollow steel strut, 2.4 m long, is pin-jointed at the ends. It has an outer diameter of 40 mm and a thickness of 5 mm. If the yield stress is 320 N/mm2, =1/7500 and E = 2 x l05 N/mm2. Compare the crippling load given by Euler’s and Rankine’s formulae. | 10M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |
| 9) | A rectangular section of breadth 300 mm and depth 500 mm carries an eccentric load of 150 kN at an eccentricity of 100 mm in a plane bisecting the depth. Determine the maximum and minimum intensities of stresses developed. | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 10) | Determine the diameter of a bolt which is subjected to an axial pull of 12 kN together with a transverse Shear Force of 6 kN using (i) Maximum principal stress theory and (ii) Maximum principal strain theory. Take the elastic limit in tension as 225 N/mm2, factor of safety as 3 and Poisson’s ratio as 0.3. | 10M | CO-4 | BL-3 |

****