| Subject Code 22PC1CE205 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular and Supplementary Examinations, May 2026

Structural Analysis

(CE)

Time: 3 hours                                                                  			 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | State the conditions for a perfect frame and distinguish it from a deficient frame. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | What is the double integration method? State its limitations. | 2M | CO-2 | BL-1 |
|  | Differentiate between a cantilever beam and a propped cantilever beam with a neat sketch. | 2M | CO-3 | BL-1 |
|  | Write any two advantages of Continuous Beams. | 2M | CO-3 | BL-1 |
|  | Define an influence line diagram. | 2M | CO-4 | BL-1 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | Determine the forces in all the members of the truss by method of joints shown in Figure and indicate the magnitude and nature of the forces on the diagram of the truss. | 10M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2 | A truss is loaded and supported as shown in Figure. Determine the axial forces in the members CD, BD, CE, EG, EF & DF. | 10M | CO-1 | BL-4 |
| UNIT-II |  |  |  |  |
| 3 | A simply supported beam of span 6 m carries a point load of 50 kN at 2 m from the left end. If E = 200 GPa and I = 40 × 10⁻⁶ m⁴, find (i) slopes at both supports, (ii) deflection under the load, and (iii) maximum deflection and its location, using Macaulay's method. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4 | Using the moment area method, determine the slope and deflection at the free end of the cantilever beam shown in figure. Take EI = 8 × 10³ kN·m². | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5 | Using the consistent deformation method, find the prop reaction, draw the SFD and BMD for the propped cantilever beam shown in figure. | 10M | CO-3 | BL-4 |
| OR |  |  |  |  |
| 6 | A fixed beam AB of span 6 m carries a concentrated load of 90 kN at 2 m from A. Determine the fixed-end moments, support reactions, and draw the SFD and BMD. | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7 | Using Clapeyron's theorem of three moments, find the support moments and draw the SFD and BMD for the continuous beam shown in figure. Take EI constant. | 10M | CO-3 | BL-4 |
| OR |  |  |  |  |
| 8 | A three-span continuous beam ABCD has spans AB = BC = CD = 4 m and is simply supported at all supports. The beam carries a UDL of 15 kN/m on span AB, a central concentrated load of 50 kN on BC, and a UDL of 10 kN/m on CD. Apply the theorem of three moments to find all support moments and SFD and BMD. Take EI constant. | 10M | CO-3 | BL-3 |
| UNIT-V |  |  |  |  |
| 9 | A simply supported girder of span 20 m is traversed by a uniformly distributed rolling load of 30 kN/m and length 8 m. Calculate (i) maximum positive and negative shear force at a section 3m from the left support and (ii) also calculate the absolute maximum bending moment in the girder. | 10M | CO-4 | BL-4 |
| OR |  |  |  |  |
| 10 | Two-wheel loads of 80 kN and 50 kN separated by 3 m cross a simply supported girder of span 12 m from left to right, with the 50 kN load leading. Calculate the maximum shear force and bending moment at a section 4 m from the left support. | 10M | CO-4 | BL-4 |

****