| Subject Code 22PC1AE207 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular and Supplementary Examinations, May 2026

Theory of Machines and Mechanisms

(AE)

Time: 3 hours                                                                        			 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | What is the significance of degrees of freedom of a kinematic chain when it functions as a mechanism? Give examples | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Define Kennedy’s theorem. | 2M | CO-1 | BL-2 |
|  | Describe the advantages of roller follower over knife-edge follower? | 2M | CO-3 | BL-2 |
|  | What do you understand by the term ‘interference’ as applied to gears? | 2M | CO-3 | BL-1 |
|  | What is the necessity of a governor? | 2M | CO-4 | BL-1 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | - Distinguish between a machine and a mechanism. | 5M | CO-1 | BL-2 |
|  | - Define kinematic pair. Distinguish between closed and unclosed type kinematic pairs. Give examples for each type. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
| 2 | Write the inversions of a slider crank chain with neat sketches. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |  |
| 3 | - Derive an expression for the magnitude and direction of coriolis component of acceleration | 5M | CO-1 | BL-3 |
|  | - Discuss the three types of instantaneous centers in a mechanism | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
| 4 | The crank of a slider crank mechanism rotates clockwise at a uniform speed of 250 rpm. The crank is 200 mm and connecting rod is 750 mm long. Find the following at crank angle of 45° from IDC: - Linear velocity of the slider - Linear velocity of the midpoint of connecting rod - Angular velocity of the connecting rod - Position and velocity of a point E on the connecting rod having least velocity relative to crank shaft | 10M | CO-1 | BL-3 |
| UNIT-III |  |  |  |  |  |
| 5 | - Explain the terms precession and gyroscopic acceleration. | 5M | CO-2 | BL-2 |
|  | - Explain the gyroscopic effects on four wheeled vehicles. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |  |
| 6 | The following data relate a cam profile in which the follower moves uniform acceleration and deceleration during accent and decent - Minimum radius of the cam =25mm - Roller diameter =7.5mm - Lift =28mm - Offset of follower axis =12mm towards right - Angle of accent=600 - angle of descent =900 - angle dwell between accent and decent =450 - speed of the cam =100rpm Draw the profile of the cam and determine the maximum velocity and uniform acceleration of the follower during the outstroke and the return stroke | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |  |
| 7 | - Define i)module ii)Pitch circle and iii)Addendum of a gear. | 5M | CO-3 | BL-2 |
|  | - Derive the expression for length of path of contact. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
| 8 | An epicyclic gear consists of three gears A, B and C as shown in Fig.3. The gear A has 72 internal teeth and gear C has 32 external teeth. The gear B meshes with both A and C and is carried on an arm EF which rotates about the centre of A at 18 r.p.m.. If the gear A is fixed, determine the speed of gears B and C. [12M] Fig.3. | 10M | CO-3 | BL-4 |
| UNIT-V |  |  |  |  |  |
| 9 | - Define sensitivity and hunting of governors. | 5M | CO-4 | BL-2 |
|  | - Calculate the vertical height of a Watt governor when it rotates at 60 r.p.m. Also find the change in vertical height when its speed increases to 61 r.p.m. | 5M | CO-4 | BL-4 |
| OR |  |  |  |  |  |
| 10 | The arms of a Porter governor are each 25 cm long and pivoted on the governor axis. Mass of each ball is 5 kg and the mass of the central sleeve is 30 kg. The radius of rotation of the balls is 15 cm when the sleeve begins to rise and reaches a value of 20 cm for maximum speed. Determine speed range of the governor. If the friction of the sleeve is equivalent to 20 N of load, determine how the speed range is modified. | 10M | CO-4 | BL-4 |

****