| Subject Code 22OE1EC307 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular and Supplementary Examinations, May/June 2026

5G/6G Technologies

(Open Elective)

Time: 3 hours                                                                        				Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A				5X2=10M

|  | Describe the technological limitations of 4G that motivated the development of 5G. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Explain the concept of Service-Based Architecture (SBA) in 5G core. | 2M | CO-2 | BL-2 |
|  | Outline about scalable numerology in 5G NR? | 2M | CO-3 | BL-2 |
|  | State the significance of beamforming in Massive MIMO systems. | 2M | CO-4 | BL-2 |
|  | Mention two enabling technologies expected to play a key role in 6G networks. | 2M | CO-5 | BL-1 |

PART- B				5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Explain the major architectural transformations introduced in 5G compared to previous generations. | 5M | CO-1 | BL-2 |
|  | - Analyze how latency reduction is achieved in 5G networks through protocol enhancements. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2 | Discuss about 5G service categories and evaluate their performance requirements with suitable justification. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | - Explain the functional architecture of 5G Core network with neat diagram. | 5M | CO-2 | BL-2 |
|  | - Analyze the role of AMF and SMF in mobility and session management. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4 | a) Discuss network slicing in 5G and infer how logical isolation and QoS differentiation are achieved. | 5M | CO-2 | BL-2 |
|  | b) With block diagram brief on 5G RAN architecture. | 5M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |
| 5 | - Describe 5G NR frame structure including slots and symbols. | 5M | CO-3 | BL-2 |
|  | - Analyze how different subcarrier spacings affect latency and coverage. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6 | a) Explain 5G frequency range classification and their impact on bandwidth and throughput. | 5M | CO-3 | BL-3 |
|  | b) Brief on symbol, slot,subframe,frame. | 5M | CO-3 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7 | - Extend the working principle of OFDM in 5G systems. | 5M | CO-4 | BL-2 |
|  | - Analyze propagation characteristics at millimeter-wave frequencies. | 5M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 8 | a) Summarize the basic MIMO system model and the benefits of Massive MIMO in dense deployments. | 5M | CO-4 | BL-2 |
|  | b) Illustrate the following concepts-small cells , D2D ,M2M. | 5M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |
| 9 | - Discuss limitations of 5G that motivate evolution toward 6G. | 5M | CO-5 | BL-3 |
|  | - Explain expected spectrum utilization and data rate targets in 6G. | 5M | CO-5 | BL-2 |
| OR |  |  |  |  |
| 10 | a) Apply OTFS modulation to counter delay and Doppler effect. | 5M | CO-5 | BL-3 |
|  | b) Analyze the role of Rate-Splitting Multiple Access (RSMA) in 6G systems. | 5M | CO-5 | BL-3 |

****