| Subject Code 22PC1EC306 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular and Supplementary Examinations, May/June 2026

CMOS VLSI DESIGN

(ECE)

Time: 3 hours                                                                        			 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | Define threshold voltage of MOSFET. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What is Common Mode Rejection Ratio(CMRR)? | 2M | CO-2 | BL-2 |
|  | Draw the two input nMOS NAND gate. | 2M | CO-3 | BL-2 |
|  | Differentiate between contact- cut and via. | 2M | CO-4 | BL-3 |
|  | List any four fault models. | 2M | CO-5 | BL-1 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Explain the basic steps involved in CMOS IC fabrication using p-well process. | 8M | CO-1 | BL-2 |
|  | - A nMOS transistor has the following parameter μn​Cox= 50μA/V2, W=20μm, L=2μm, VGS ​= 4V, VT​=1V, VDS ​= 2V, Determine the region of operation and calculate the drain current IDS? | 2M | CO-1 | BL-4 |
| OR |  |  |  |  |
| 2 | Analyze the small-signal model of a MOSFET including channel length modulation and body bias effect? | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | Describe the design and working of a basic differential amplifier? | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4 | Explain the operation of a simple current mirror with neat diagram. Derive the expression for output current and discuss the effect of channel length modulation? | 10M | CO-2 | BL-4 |
| UNIT-III |  |  |  |  |
| 5 | Derive the Voltage Transfer Characteristic (VTC) of a CMOS inverter? Sketch the VTC and clearly mark VOH, VOL, VIH, VIL, and switching threshold. | 10M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6 | What ae the advantages of CMOS logic over nMOS logic? Realize the following Boolean expression using CMOS logic: Y = A+BCD | 10M | CO-3 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7 | Draw the stick diagram and layout of an NMOS inverter? | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 8 | a) What are λ-based design rules? Explain the design rules of wires and transistors. | 5M | CO-4 | BL-2 |
|  | b) Explain any five scaling factors of MOSFET. | 5M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |
| 9 | - Explain the need for testing in VLSI circuits. Discuss the role of testing in manufacturing? | 5M | CO-5 | BL-2 |
|  | - Explain about stuck at faults. | 5M | CO-5 | BL-2 |
| OR |  |  |  |  |
| 10 | Discuss Built-In Self-Test (BIST) architecture in detail? | 10M | CO-5 | BL-2 |

****