| Subject Code 22PE1EE302 |  | R22 |
| --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular and Supplementary Examinations, May 2026

ELECTRIC DRIVES

(EEE)

Time:3 hours                                                                        			 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | Draw the different types of load torques curves? | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | What are the advantages of chopper fed DC drives. | 2M | CO-2 | BL-1 |
|  | What are the limitations of v/f ratio control. | 2M | CO-3 | BL-1 |
|  | What are the advantages of slip power recovery scheme? | 2M | CO-4 | BL-2 |
|  | What are the applications of BLDC motor drive? | 2M | CO-5 | BL-2 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | A 220V, 1200 rpm, and 10A separately excited dc motor is fed from a single phase full converter with ac source voltage of 230V, 50Hz. Armature circuit resistance is1Ω. Armature current is continuous. Calculate firing angle for - rated motor torque at 600 rpm ii) half the rated motor torque at (-600) rpm | 10M | CO-1 | BL-2, BL-3 |
| OR |  |  |  |  |
| 2 | Explain the operation of separately excited DC motor when fed from single phase full controlled rectifier for discontinuous mode of operation. | 10M | CO-1 | BL-2, BL-3 |
| UNIT-II |  |  |  |  |
| 3 | A 230V, 960 r.p.m, 200A separately excited DC motor has an armature resistance of 0.02Ω. The motor is fed from a chopper for both motoring and braking operations. The source has a voltage of 230V, assuming continuous conduction mode. a) calculate duty ratio of the chopper for motoring operation at rated torque and 350 r.p.m b) calculate duty ratio of the chopper for braking operation at rated torque and 350 r.p.m. | 10M | CO-2 | BL-2, BL-3 |
| OR |  |  |  |  |
| 4 | Discuss the regenerative braking and plugging of a DC shunt motor. | 10M | CO-2 | BL-2, BL-3 |
| UNIT-III |  |  |  |  |
| 5 | Explain the V/f ratio control of Induction motor drive. | 10M | CO-3 | BL-2, BL-3 |
| OR |  |  |  |  |
| 6 | A 2.8kW, 4-pole, 400V, 50Hz, 1370 r.p.m delta connected induction motor drive has the following parameters when referred to stator. Rs=2Ω, Rr¦=5Ω, Xs= Xr¦=5Ω, Xm = 80Ω.Motor speed is controlled by stator voltage control. When driving a fan load it runs at rated voltage. Calculate motor terminal voltage, current and torque at 1200 r.p.m. | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7 | Explain in detail about rotor resistance control of Induction Motor. | 10M | CO-4 | BL-2, BL-3 |
| OR |  |  |  |  |
| 8 | Discuss the static scherbius drive in detail. | 10M | CO-4 | BL-2, BL-3 |
| UNIT-V |  |  |  |  |
| 9 | Discuss the self-controlled mode operation of synchronous motor drive. | 10M | CO-5 | BL-2, BL-3 |
| OR |  |  |  |  |
| 10 | Explain the operation of BLDC motor for 180 degree switching strategy of operation. | 10M | CO-5 | BL-2, BL-3 |

***