| Subject Code 22PC1ME306 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular and Supplementary Examinations, May/June 2026

FINITE ELEMENT METHODS

(ME)

Time: 3 hours                                                                       			 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | Define shape function and write down the shape function for one dimensional bar element. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Distinguish between beam and truss elements with a neat sketch. | 2M | CO-1 | BL-1 |
|  | Draw a tetrahedron element with degrees of freedom for structural analysis. | 2M | CO-2 | BL-1 |
|  | List the boundary conditions of two dimensional steady state heat transfer problems. | 2M | CO-3 | BL-1 |
|  | Define lumped mass matrix. | 2M | CO-4 | BL-1 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Summarize the basic steps involved in finite element analysis. | 5M | CO-1 | BL-2 |
|  | - Derive the strain displacement [B] matrix for a 2 noded uniaxial bar element. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2 | A steel bar of length 800mm is subjected to an axial load of 3kN as shown in figure. Evaluate the nodal displacement of the bar at 400mm and 800mm respectively. Find the stress in the elements. Take E= 2 x 105N/mm2. Area of the bar is 800 mm2.Consider as a two element system. | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3 | Compute the displacements at nodes in elements shown in figure. Take E= 70000 N/mm2 and A=200 mm2. | 10M | CO-1 | BL-4 |
| OR |  |  |  |  |
| 4 | Derive Hermite shape functions for a beam element and state the significance. | 10M | CO-1 | BL-3 |
| UNIT-III |  |  |  |  |
| 5 | - Derive strain displacement matrix for triangular CST element and formulate the element stiffness matrix [k]. | 5M | CO-2 | BL-3 |
|  | - The (x, y) coordinates of the nodes i, j and k of a triangular element are (1,1), (4, 2) and (3, 5) respectively. The shape functions of a point P located inside the element are given by N1 = 0.15 and N2 = 0.25. Determine the x and y coordinates of the point P. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 6 | An axi-symmetric ring element is shown in figure. Evaluate the matrices [B] and [D]. Take E = 2 X 105 N / mm2 and μ = 0.33.Formulate stiffness matrix. | 10M | CO-2 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7 | A metallic fin with thermal conductivity k=360 W /m 0C, 0.001 m thick and 0.1m long, extends from a plane wall whose temperature is 2350C. Determine the temperature distribution and amount of heat transferred from the fin to the air at 200C with h= 9 W/m2C. Take the width of fin to be 1 m. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 8 | A composite wall consists of 4 cm thick wood, 10 cm glass fiber insulation and 1 cm thick plaster. If the temperature on wood and plaster faces are 200C and -200C respectively. Determine the temperature distribution in the wall. Assume the thermal conductivity of wood, glass fiber and plaster are 0.17, 0.035 and 0.5 W/m K respectively and colder side heat transfer coefficient is 25 W/m2 K. | 10M | CO-3 | BL-3 |
| UNIT-V |  |  |  |  |
| 9 | - Derive the elemental consistent mass matrix for 1-D bar element. | 5M | CO-4 | BL-3 |
|  | - Evaluate the integral by two point gauss quadrature rule. | 5M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 10 | Consider axial vibration of the steel bar shown in Figure. Determine the natural frequencies and Eigen vectors. Take E=210GPa.Density =8000 kg/m3. | 10M | CO-4 | BL-4 |

****