| Subject Code 22PC1ME304 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular and Supplementary Examinations, May 2026

HEAT TRANSFER

(ME)

Time: 3 hours                                                                        			 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | What are the different modes of heat transfer? Mention one example for each mode. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | What is overall heat transfer coefficient? | 2M | CO-1 | BL-1 |
|  | What is the significance of Nusselt number? | 2M | CO-2 | BL-2 |
|  | Distinguish between drop-wise and film-wise condensation. | 2M | CO-2 | BL-2 |
|  | Define solid angle & radiation shape factor | 2M | CO-3 | BL-1 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - State and explain the Fourier’s law of heat conduction. Write formula with units of components. | 5M | CO-1 | BL-1 |
|  | - Write short notes on thermal diffusivity. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2 | Derive general heat conduction equation in cylindrical co-ordinates. | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3 | - What do you understand by characteristic length used in lumped heat analysis? | 3M | CO-1 | BL-2 |
|  | - An Aluminum fin of 7 mm thick and 50 mm long protrudes from a wall, which is maintained at 120oC. The ambient air temperature is 22oC. The heat transfer coefficient and conductivity of the fin material are 140 W/m2K and 55 W/mK respectively. Determine i) Temperature at the end of the fin, ii) Total heat dissipated by the fin. | 7M | CO-1 | BL-4 |
| OR |  |  |  |  |
| 4 | Calculate the rate of heat flow through a plane homogeneous wall of thickness 50 mm made from i) Steel (k = 40 W/mK) and ii) Concrete (k = 1.1 W/mK). The temperature of the wall surfaces are maintained at 100oC and 90oC. Assume that the thickness is considerably smaller than its width and height. For the same amount of heat transfer as in concrete, how much thickness is to be made for steel. | 10M | CO-1 | BL-4 |
| UNIT-III |  |  |  |  |
| 5 | - Sketch the hydrodynamic and thermal boundary layer development of a flow over flat plate and explain the significance of the boundary layer | 5M | CO-2 | BL-2 |
|  | - What is Dittus Boeltier equation? Where and when does it apply? | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 6 | Calculate the average coefficient of heat transfer for natural convection for a vertical plate 30.48 cm high at 51.67oC. The surrounding air is at 23.9oC. Also calculate the boundary layer thickness at the trailing edge of plate. | 10M | CO-2 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7 | - What is boiling & evaporation? | 2M | CO-2 | BL-2 |
|  | - Explain the different stages of pool boiling curve with the help of neat sketch. | 8M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 8 | A heat exchanger heats 25,000 kg/hr of water entering at 30oC while cooling 20,000 kg/hr of water from 100oC to 80oC =. Determine the area necessary for i) parallel flow arrangement, ii) Counter flow arrangement. Overall heat transfer coefficient may be assumed as 1,600 W/m2K. | 10M | CO-2 | BL-4 |
| UNIT-V |  |  |  |  |
| 9 | - Define the following terms i) Solid angle, ii) Shape factor, iii) Radiosity iv) Emissivity, v) Gray body | 5M | CO-3 | BL-2 |
|  | - Two very large parallel plates with emissivities 0.5 exchange heat. Determine the percentage reduction in the heat transfer rate if a polished aluminum radiation shield of ε = 0.04 is placed in between the plates. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 10 | Two parallel square plates each 4 m2 area are large compared to a gap of 5 mm separating them. One plate has a temperature of 800 K and surface emissivity of 0.6, while the other has temperature of 300 K and surface emissivity of 0.9; find the net exchange by radiation between the plates. If a thin polished metal sheet of surface emissivity 0.1 on both sides is now located centrally between the two plates, what will be its steady state temperature? | 10M | CO-3 | BL-4 |

****