| Subject Code 22OE1EC306 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular and Supplementary Examinations, May/June 2026

Programmable Logic Devices for Microelectronics

(Open Elective)

Time: 3 hours                                                                        			 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | Give the differences between latch and flip-flop. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What are the advantages and disadvantages of using a PROM as a PLD? | 2M | CO-2 | BL-1 |
|  | What is a Look-Up Table (LUT) in an FPGA? | 2M | CO-3 | BL-1 |
|  | What is a "Bit stream"? | 2M | CO-4 | BL-1 |
|  | List general design issues in programmable logic devices. | 2M | CO-5 | BL-1 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Explain the operation JK master slave flip flop. Explain its truth table. | 5M | CO-1 | BL-2 |
|  | - Explain the realization of SR flip-flop, JK flip-flop using D flip-flop. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2 | Develop Logic diagram and Truth Table, discuss 8×1 Multiplexer and then realize f (x, y, z)m 1,2,4,7 using 8×1 MUX as well as using 4×1 MUX. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | - Demonstrate the following Boolean functions using PAL F1 (X, Y, Z) = ∑ (1, 2, 4, 6) F2 (X, Y, Z) = ∑ (0, 1, 6, 7) F3 (X, Y, Z) = ∑ (2, 6) | 5M | CO-2 | BL-2 |
|  | - What are the merits & demerits of PROM, PAL, PLA | 5M | CO-2 | BL-1 |
| OR |  |  |  |  |
| 4 | Contrast a combinational circuit using a ROM. The circuit accepts a 3 bit number and generates an output binary number equal to the square of the input number | 10M | CO-2 | BL-4 |
| UNIT-III |  |  |  |  |
| 5 | - Compare and contrast Functional Simulation and Gate-Level Simulation (GLS) | 5M | CO-3 | BL-1 |
|  | - Explain the complete Synthesis Flow in detail with a block diagram. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6 | Explain the general internal architecture of an FPGA with a neat block diagram. Discuss the function of each component | 10M | CO-3 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7 | - Demonstrate the features, advantages and applications of FPGA. | 5M | CO-4 | BL-2 |
|  | - Identify different FPGA programming technologies and explain any one. | 5M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 8 | Draw and explain FPGA implementation flow including simulation and synthesis. | 10M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |
| 9 | - Explain the internal structure of a macrocell in CPLD | 5M | CO-5 | BL-2 |
|  | - Explain the ACT2 architecture for high fan-in example? | 5M | CO-5 | BL-2 |
| OR |  |  |  |  |
| 10 | Analyze the design and implementation of a Full Adder using FPGA | 10M | CO-5 | BL-4 |

******