| Subject Code 22PE1ME306 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular and Supplementary Examinations, May 2026

REFRIGERATION AND AIR CONDITIONING

(ME)

Time: 3 hours                                                                        			 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | Define tonne of refrigeration. | 2M | CO-1,2 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Discuss the ideal properties of refrigerant and absorbent combination. | 2M | CO-1,2 | BL-2 |
|  | Draw the p-v and T-s diagrams of Bell-Coleman cycle. | 2M | CO-2,3 | BL-2 |
|  | With the help of psychometric chart represent humidification and sensible cooling processes. | 2M | CO-3,4 | BL-2 |
|  | What are the different heat sources in a room. | 2M | CO-3,4 | BL-2 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Discuss the essential properties of good refrigerant. Give the examples of commonly used refrigerants. | 5M | CO-1,2 | BL-2 |
|  | - Explain the working of Vapour compression refrigeration cycle. | 5M | CO-1,2 | BL-2 |
| OR |  |  |  |  |
| 2 | A vapour compression refrigeration cycle works between temperature limits of 250C and -100C. If the gas is dry at the end of compression, calculate COP of the system. Assuming no under cooling of the liquid ammonia. Use the following table for the properties of ammonia. Temperature (0C); Liquid heat (kJ/kg); Latent heat (kJ/kg); Liquid Entropy (kJ/kg K). 25; 298.9; 1166.94; 1.1242. -10; 135.3; 1297.68; 0.5443. | 10M | CO-1,2 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | Explain the working of lithium bromide and water absorption refrigeration system with a neat sketch. List the applications and advantages of it. | 10M | CO-1,3 | BL-2 |
| OR |  |  |  |  |
| 4 | Explain the working of thermo electric refrigeration system. What are the merits and demerits over the other refrigeration systems? | 10M | CO-1,3 | BL-2 |
| UNIT-III |  |  |  |  |
| 5 | a) A refrigerator working on Bell-Coleman cycle operates between pressure limits of 1.05 bar and 8.5 bar. Air is drawn from the cold chamber at 100C, compressed and then it is cooled to 300C before entering the expansion cylinder. The expansion and compression follows the law pv1.3=constant. Determine the theoretical C.O.P. of the system. | 5M | CO-2,3 | BL-3 |
|  | b) Give the comparison between open and dense air refrigeration systems. | 5M | CO-2, 3 | BL-2 |
| OR |  |  |  |  |
| 6 | Give the classification of evaporators used in the refrigeration systems. Explain the working of flood type evaporator with neat sketch. | 10M | CO-2,3 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7 | The readings from a sling psycho meter are as follows, Dry bulb temperature = 300 C; Wet bulb temperature = 200 C; Barometer reading = 740 mm of Hg. Determine 1. Dew point temperature; 2. Relative humidity; 3. Specific humidity; 4. Degree of saturation; 5. Vapour density; 6. Enthalpy of mixture per kg of dry air. | 10M | CO-2,3 | BL-3 |
| OR |  |  |  |  |
| 8 | Explain the working of summer air conditioning system with a neat sketch. | 10M | CO-3,4 | BL-2 |
| UNIT-V |  |  |  |  |
| 9 | Explain the different types of heat pump circuits. Describe any one circuit in detail with neat diagram. | 10M | CO-3,4 | BL-2 |
| OR |  |  |  |  |
| 10 | What do you mean by human comfort? Explain factors which affect the human comfort. Explain the concept of effective temperature. | 10M | CO-3,4 | BL-2 |

****