| Subject Code 22PE1ME310 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year II Semester Regular and Supplementary Examinations, May 2026

THEORY OF METAL CUTTING

(ME)

Time: 3 hours                                                                        			 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | Differentiate between orthogonal cutting and oblique cutting. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What are types of tool wear | 2M | CO-2 | BL-2 |
|  | List out types of tool materials | 2M | CO-3 | BL-3 |
|  | Write any two properties of cutting fluids | 2M | CO-4 | BL-4 |
|  | List the elements of total production cost | 2M | CO-4 | BL-4 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | What are the types of chips formed in metal cutting, explain with relevant diagrams. | 10M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2 | Derive the various cutting forces using Merchant circle diagram. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | - What are the reasons for measuring the cutting forces. | 5M | CO-2 | BL-1 |
|  | - Explain the tool life criteria and machinability index | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 4 | Discuss various types of dynamometers used for force measurement during metal cutting | 10M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |
| 5 | - Write the applications of single point cutting tools in orthogonal cutting. | 5M | CO-2 | BL-1 |
|  | - write the characteristics of tool materials. | 5M | CO-2 | BL-1 |
| OR |  |  |  |  |
| 6 | Describe the tool angles specifications in ISO system with neat sketch. | 10M | CO-2 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7 | - Discuss the temperature distribution in metal cutting. | 5M | CO-3 | BL-3 |
|  | - Describe the selection of cutting fluids. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 8 | Explain the experimental distribution of tool temperatures. | 10M | CO-3 | BL-3 |
| UNIT-V |  |  |  |  |
| 9 | Explain the elements of total production cost in detail. | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 10 | Derive the equations for optimal cutting speed, tool life for minimum cost | 10M | CO-4 | BL-3 |

****